| C:L | P,T | Variable Name |  | Meaning (units) | Notes |
| --- | --- | --- | --- | --- | --- |
| 8:1 | 1-80, A | WHAT |  | “DATA reduction parameters are next” | See Section III.E.3.b for a discussion of data-reduction parameters. This card set could be put into the INPut file rather than PARameter file if no values are to be varied. |
| 8:2 | 1-5, A | NAMDTP |  | Alphanumeric name for the first data-reduction parameter |  |
|  | 7, I | Iff |  | Flag to indicate how this parameter is to be treated | 1 if varied 3 if PUP (see Section IV.D.2) 0 otherwise |
|  | 11-20, F | PARDTP |  | Current value of parameter |  |
|  | 21-30, F | UNCDTP |  | Absolute uncertainty for this parameter |  |
|  | 31-40, F | PTILDE |  | Value of parameter used to generate partial derivatives | If zero, SAMMY assumes PTILDE = PARDTP |
| 8:5,6 etc. Repeat lines 2,3,4... as required |  |  |  |  |  |
| 8:Last (blank) |  |  |  |  |  |