VI.F.4.  Format of the MXW File

In order to generate stellar (Maxwellian) averages of capture cross sections (see Section V.D), two pieces of input are needed.  In the alphanumeric portion (card set 3) of the INPut file, the line

MAXWELLIAN AVERAGED capture cross sections are wanted

or

STELLAR AVERAGED CAPture cross sections are wanted

must be included (the two phrases are treated identically within the code).  It is also necessary to specify the temperatures at which the stellar averages are to be calculated; these are given in a “MXW” file whose name is specified immediately following the name of the DATa file in the input stream.  The format of the MXW file is one temperature per line, in F10.1 format, in units of eV.

When calculating stellar averages, it is often desirable to include cross sections at energies above the resolved resonance region.  In SAMMY, this may be done by providing the “smooth cross section” in the ENDF File 3 format.  Users must also include the following line

ADD CROSS SECTIONS From endf/b file 3

in the alphanumeric portion of the INPut file.

For users unfamiliar with ENDF formats, a description of the relevant portions of File 3 is presented in Table VI F4.1.  Note that the parameter INT specifies an interpolation scheme for NBT energy points.  For INT = 1, the cross section from Ei to Ei+1 is constant and equal to Ci.  For INT = 2, the cross section for Ei < E < Ei+1 is linearly interpolated between the two values.  (Note that other options are available in ENDF but only these two have been implemented in SAMMY.)

Examples for stellar averaging are given in test cases tr042, tr049, and tr051.

Table VI F4.1.  Format for File 3 cross sections to be added for stellar averages

A decimal point must occur somewhere with an F format

| Line Number | Variable | Col-umns | Format | Notes |
| --- | --- | --- | --- | --- |
| 1 | (Nothing on this line is important to SAMMY runs) |  |  |  |
| 2 | NR | 45-55 | I11 | Number of energy regions |
|  | NP | 56-66 | I11 | Total number of energy points to be specified |
| 3 | NBT(1) | 1-11 | I11 | Number of energy points in region 1 |
|  | INT(1) | 12-22 | I11 | Interpolation type for region 1 (INT=1 implies constant, INT=2 implies linear; contact the SAMMY author if other options are needed.) |
|  | NBT(2) | 23-33 | I11 | Number of energy points in region 2 |
|  | INT(2) | 34-44 | I11 | Interpolation type for region 2 |
|  | NBT(3) | 45-55 | I11 | Number of energy points in region 3 |
|  | INT(3) | 56-66 | I11 | Interpolation type for region 3 |
| 4,5, ... | Repeat Line 3 until all NR regions have been specified. Note that the sum of the NBT values must equal NP. |  |  |  |
| 6 | E(1) | 1-11 | F11 | First energy value |
|  | C(1) | 12-22 | F11 | Value of cross section at energy E(1) |
|  | E(2) | 23-33 | F11 | Next energy value |
|  | C(2) | 23-44 | F11 | Value of cross section at energy E(2) |
|  | E(3) | 45-55 | F11 | Next energy value |
|  | C(4) | 56-66 | F11 | Value of cross section at energy E(3) |
| 7,8,... | Repeat Line 6 until all NP energy points have been specified |  |  |  |