| C:L | P,T | Variable Name | Meaning (units) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 11:1 | 1-80, A | WHAT | “MISCEllaneous parameters follow” |  | Any line in this card set may be omitted when not needed. This card set may be in INPut file. |
| 11:2 | 1-5, A | WHAT | “DELTA” |  | is used instead of DELTAL from card set 4 of this file, or instead of DELTAL from card set 5 of the INPut file. Flags = 1 to vary, 3 to PUP, and 0 to hold fixed. |
|  | 7, I | IFLAG1 | Flag to indicate treatment for parameter |  |  |
|  | 9, I | IFLAG0 | Flag to indicate treatment for parameter |  |  |
|  | 11-20, F | DELL11 | = Coeff. of E (m/eV) |  |  |
|  | 21-30, F | D1 | Uncertainty on (m/eV) |  |  |
|  | 31-40, F | DELL00 | = Constant term (m) |  |  |
|  | 41-50, F | D0 | Uncertainty on (m) |  |  |
| 11:3 | 1-5, A | WHAT | “ETA ” (“eta” plus two blank spaces) |  | Use if type of data (see card set 8 of INput file) is ETA. |
|  | 7, I | IFLAGN | Flag to indicate treatment for parameter |  |  |
|  | 11-20, F | NU | Normalization coefficient |  | NU = (dimensionless) as in ETA = NU  (fission /absorption) |
|  | 21-30, F | DNU | Uncertainty on NU |  |  |
|  | 31-40, F | ENU | Energy for which this value of NU applies (eV) |  | If a constant value of NU is wanted, the energy value can be omitted. If more than one ETA line is present, all must be together in the file and in increasing energy order. SAMMY will linearly interpolate to obtain the value of NU between the specified energies. |
| 11:4 | 1-5, A | WHAT | “FINIT” |  | Finite-size corrections for angular distributions; See Section III.E.7. |
|  | 7, I | IFLAGI | Flag to indicate treatment for parameter ATTNI |  | Repeat this line once for each angle. If the line occurs only once, the same attenuations are used for each angle. |
|  | 9, I | IFLAGO | Flag to indicate treatment for parameter ATTNO |  |  |
|  | 11:20, F | ATTNI | Incident-particle attenuation (atoms/barn) | Flags = 1 to vary, 3 to PUP, 0 to hold fixed. |  |
|  | 21:30, F | DTTNI | Uncertainty on ATTNI |  |  |
|  | 31:40, F | ATTNO | Exit-particle attenuation (atom/b) |  |  |
|  | 41:50, F | DTTNO | Uncertainty on ATTNO |  |  |