| C:L | P,T | Variable name | Meaning (units) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 8:2 | 1- 5, I | NANGLE | Number of angles |  | This line and those following are present only for data types “DIFFErential elastic cross sections” or “DIFFErentiAL REaction.” See Section II.B.1.b for details. |
|  | 11-20, F | ANGLE(1) | First angle, in laboratory; between 0 and 180 degrees |  |  |
|  | 21-30, F | ANGLE(2) | Second angle (degrees) |  |  |
|  | ... |  |  |  |  |
|  | 71-80, F | ANGLE(7) | Seventh angle |  |  |
| 8:3 etc. | 1-10, F 11-20, F etc. | ANGLE(8) ANGLE(9) etc. | Other angles, for a total of NANGLE |  | These lines are absent if NANGLE < 8. |
| 8:4 | 11-20, F | DANGLE(1) | Angular spread of detector at ANGLE(1) (degrees) |  |  |
|  | ... | DANGLE(7) | Spread at ANGLE(7) |  |  |
| 8:6 etc. | 1-10, F 11-20, F etc. | DANGLE(8) DANGLE(9) etc. | Spread at other angles, for a total of NANGLE |  | These lines are absent if NANGLE < 8. |
| 9 | This card set, which originally contained the spin of the target nuclide, is no longer needed and should be removed from INPut files. The spin of the target nuclide is now given either in card set 10.1 or in card set 4 or 4a. Currently, SAMMY will ignore this card if it is present. However, that capability will not necessarily be maintained in future releases of the code. |  |  |  |  |
| 10 | The original card set 10 is obsolete and must be replaced by either card set 10.1 or card set 10.2. To convert your old INPut files, begin a SAMMY run using that file; SAMMY will generate a file INPUT.NEW containing card set 10.1. Caution: Information in INPUT.NEW may not be entirely correct, as information in the obsolete format can be misinterpreted. It is the user’s responsibility to ensure that the conversion is correct. |  |  |  |  |