| C:L | P,T | Variable name | Meaning (units) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 5:1 | 1-10, F | TEMP | Effective temperature of the sample (K); zero for no Doppler broadening. This value may be overwritten in PARameter card set 4 (see Table VIB.2). |  | This card set is omitted when “BROADENING IS NOT WAnted” is specified in card set 3. (SAMMY will abort if this rule not followed.) |
|  | 11-20, F | DIST | Flight-path length (m) |  |  |
|  | 21-30, F | DELTAL | That portion of the Gaussian resolution width attributed to the flight-path length (m). |  | Value may be overwritten in PARameter card set 4 (see Table VIB.2). |
|  | 31-40, F | DELTAE | E-folding width of exponential resolution function (μs); (0 for pure Gaussian resolution function). This value may be overwritten in PARameter card set 4 (see Table VIB.2). |  | See Section III.C.1.b. If the line “EXPONENTIAL FOLDING width is energy-dependent” is specified in card set 3 of the INPut file, then see Eq. (III C1 b.6) for details of the energy dependence. |
|  | 41-50, F | DELTAG | The absolute value of DELTAG is the FWHM (μs) of a Gaussian resolution function representing the burst width. (DELTAG = 0 for a purely exponential resolution function.) |  | See Section III.C.1.a, Eqs. (III C1 a.13) and (III C1 a.15). If DELTAG is negative, channel widths are given in card set 6. |
|  | 51-60, F | DELTTT | Approximate step size for the Effective Temperature for the Leal-Hwang method of Doppler broadening (K). Value > 0; default = 5.0 |  | See Section III.B.2. If DELTTT < 0.0, an error message will be printed and the code will cease operation. |
|  | 61-70, F | ELOWBR | The energy below which no broadening is wanted, for high-energy Gaussian approximation to Doppler broadening (eV) |  | Default = 0.0 |
|  | 71-80, F | DDDEEE | Range for straight-line energy average resolution function (eV) |  | See Section III.C.4. |