III.C.2.   Oak Ridge Resolution Function (ORR)

For ORELA data, the resolution-broadening functions described in Section III.C.1 are not especially accurate approximations.  We have therefore developed a more realistic mathematical description of the experimental situation at ORELA, based on concepts described by D. C. Larson  et al. [DL84] and initially derived by F. G. Perey [FP89].

The resolution function has four components:  the electron burst, the ORELA moderator, the neutron detector, and the time-of-flight channel width.  Experimentally, the resolution-broadened cross section  is formed when each contributing feature provides its own broadening; the combined effect of the several components is all that is actually noticed.  Mathematically, this may be viewed as follows:  Each component is modeled separately as a function of flight time (rather than energy), and then each is convoluted in turn with the unbroadened (or partially broadened) cross section; the convolution process is performed analytically where possible and numerically where necessary.  That is, the resolution-broadened cross section  may be expressed as

where Ii is our mathematical model for the ith component.  This expression may then be rearranged into the form

where the resolution function  is defined as

Because SAMMY deals with cross sections as functions of energy, rather than time, the relationship

is used to convert from energy to time (or time to energy) in Eq. , yielding

In the following sections we describe the components Ii and the convolution procedure.  Input for using this resolution function is described in card set 9, Table VIB.2.  See also Section X.H, which describes program SAMORT, used for plotting the individual pieces and the complete ORR function at a specified energy.