X.R.	SAMGY2:  SMOOTH THE TABULATED Y2 FUNCTION

The program SAMGY2 is used to read Y2 values printed by the Monte-Carlo code SAMSMC into the file MC_2.DAT, smooth the tabulated Y2 function, and print into a file called SAMY2.DAT. This output file can then be used for input to a SAMMY run using the command

Y2 VALUES ARE TABULAted  ;

values from that file are used instead of the SAMMY-generated Y2 values in situations where the SAMMY-generated values are insufficiently accurate.  Details for generation and use of the SAMY2.DAT file are given in Section III.D and in [NL08].

Because the values produced by Monte-Carlo calculations contain noise, it is advantageous to smooth the curve before using it in the SAMMY multiple-scattering correction procedure.  The algorithm for that smoothing is unsophisticated but could easily be improved if the need arises.  The current algorithm is an unweighted average over seven nearest-neighbor values,

Simulation sim009 gives an example of the use of code SAMGY2.