| C:L | P, T | Variable Name | Meaning (units) | Notes |
| --- | --- | --- | --- | --- |
| 14:1 | 1-80, A | WHAT | “RPI Resolution” “GEEL resolution” “GELINa resolution” “NTOF resolution” | See Section III.C.3. This card set could be put into the INPut file rather than PARameter file if no values are to be varied. |
| 14:2 | 1-5, A | WHAT | “BURST” | This line may be omitted. |
|  | 7, I | IP | Flag to vary P | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20, F | P | Full width at half max of burst (ns) |  |
|  | 21-30, F | dP | Uncertainty on P |  |
| 14:3 | 1-5, A | WHAT | “TAU ” (two spaces needed) | This line may be omitted, in which case line 4 is also omitted. |
|  | 6, I to 10, I | Iτ1 to Iτ5 | Flags to vary τ1 to τ5 one flag per column | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20, F to 71-80, F | TAU1 to TAU7 | τ1 through τ7 10 spaces for each value | These lines give parameters for τ from Eq. (III C3 a.4). |
| 14:4 | 6, I | Iτ6 | Flag to vary τ6 | This line is present if and only if line 3 is also present. |
|  | 7, I | Iτ7 | Flag to vary τ7 | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20, F to 71-80, F | dTAU1 to dTAU7 | Uncertainty on τ1 through τ7 10 spaces for each value |  |
| 14:5 | 1-5, A | WHAT | “LAMBD” | This line may be omitted, in which case line 6 is also omitted. |
|  | 6, I to 10, I | IΛ0 to IΛ4 | Flag to vary Λ0, Λ1, Λ2, Λ3, Λ4 One space per flag | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20, F to 51-60, F | LAM0 to LAM4 | Λ0, Λ1, Λ2, Λ3, Λ4 10 spaces for each value | See Eq. (III C3 a.3) and Tables III C3 b.1 and 2 for details. |
| 14:6 | 11-20, F to 51-60, F | dLAM0 to dLAM4 | Uncertainties on Λ0, Λ1, Λ2, Λ3, Λ4 | This line is present if and only if line 5 is also present. |
| 14:7 | 1-5, A | WHAT | “A1 ” (That’s “A One, blank blank blank”) | This line may be omitted, in which case line 8 is also omitted. |
|  | 6, I to 10, I | IAA1 to IAA5 | Flag to vary a1, a2, a3, a4, a5 (one space per flag) | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20, F to 71-80, F | AA1 to AA7 | Values for a1, a2, a3, a4, a5, a6, a7 from Eq. (III C3 a.5) 10 spaces for each value | See Eq. (III C3 a.5) and Tables III C3 b.1 and 2 for details. |
| 14:8 | 6, I | IAA6 | Flag to vary a6 | This line is present if and only if line 7 is also present. |
|  | 7, I | IAA7 | Flag to vary a7 |  |
|  | 11-20, F to 71-80, F | dAA1 to dAA7 | Uncertainties on a1, a2, a3, a4, a5, a6, a7 |  |
| 14:9 | 1-5, A | WHAT | “EXPON” | This line may be omitted (in which case line 10 is also omitted). See Eq. (III C3 a.2) and Tables III C3 b.1 and 2 for details. If lines 11 and 12 are present, the value for A3 given here will be ignored. Likewise, when lines 13 and 14 are present, the value for A5 given here will be ignored. |
|  | 6, I to 10, I | IT0, IA2 to IA5 | Flag to vary t0, A2, A3, A4, A5 One space per flag |  |
|  | 11-20, F to 51-60, F | T0, A2 to A5 | Values for t0, A2, A3, A4, A5 from Eq. (III C3 a.2) 10 spaces for each value |  |
| 14: 10 | 11-20, F to 51-60, F | dT0, dA2 to dA5 | Uncertainties on t0, A2, A3, A4, A5 10 spaces for each value | This line is present if and only if line 9 is also present. |
| 14: 11 | 1-5, A | WHAT | “A3SQE” or “A3 ” ( “A 3 blank blank blank”) | This line may be omitted, in which case line 12 is also omitted and A3 is equal to the value given in line 9. With WHAT=“A3SQE”, the value of α3 is sqrt(E). With WHAT=“A3 ”, α3 = 1. See Eq. (III C3 a.5) for details. |
|  | 6, I to 10, I | IAA31 to IAA35 | Flag to vary a31, a32, a33, a34, a35. One space per flag |  |
|  | 11-20, F to 71-80, F | AA31 to AA37 | Values for a31, a32, a33, a34, a35, a36, and a37 from Eq. (III C3 a.5), 10 spaces for each. |  |
| 14: 12 | 6, I | IAA36 | Flag to vary a36 | This line is present if and only if line 11 is also present. |
|  | 7, I | IAA37 | Flag to vary a37 | 0, 1, or 3 to keep, vary, or PUP. |
|  | 8, I | indicator | 2  multiply A3 by sqrt(E) 0  do not |  |
|  | 11-20, F to 71-80, F | dAA31 to dAA37 | Uncertainties on a31, a32, a33, a34, a35, a36, and a37 |  |
| 14: 13 | 1-5, A | WHAT | “A5SQE” or “A5 ” ( “A 5 blank blank blank”) See notes in next column. | This line may be omitted, in which case line 14 is also omitted and A5 is assumed to be energy independent. With WHAT=“A5SQE”, the value of α5 is sqrt(E). With WHAT=“A5 ”, α5 = 1. See Eq. (III C3 a.5) for details. |
|  | 6, I to 10, I | IAA31 to IAA35 | Flag to vary a51, a52, a53, a54, a55. One space per flag |  |
|  | 11-20, F to 71-80, F | AA51 to AA57 | Values for a51, a52, a53, a54, a55, a56, and a57 from Eq. (III C3 a.5), 10 spaces each |  |
| 14: 14 | 6, I | IAA56 | Flag to vary a56 | This line is present if and only if line 13 is also present. |
|  | 7, I | IAA57 | Flag to vary a57 |  |
|  | 8, I | indicator | 2  multiply A5 by sqrt(E) 0  do not |  |
|  | 11-20, F to 71-80, F | dAA51 to dAA57 | Uncertainties on a51, a52, a53, a54, a55, a56, and a57 |  |
| 14: 15 | 1-5, A | WHAT | “XXPON” | This line may be omitted. Line gives values for extra exponential terms Eq. (III C3 a.2). |
|  | 6, I | IB1 | Flag to vary B1 |  |
|  | 7, I | IB2 | Flag to vary B2 | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20 | B1 | Value for B1 | See Section III.C.3.a. |
|  | 21-30 | B2 | Value for B2 | If zero, lines 16 and 17 must be given. If not zero, this is the value for B2. |
|  | 31-40 | dB1 | Uncertainty for B1 |  |
|  | 41-50 | dB2 | Uncertainty for B2 |  |
| 14: 16 | 1-5, A | WHAT | “XXPON” | This line and the next are present only if B2 is zero in line 15. |
|  | 6, I to 10, I | IBI1 to IBI5 | Flag to vary bi1, bi2, bi3, bi4, bi5. One space per flag | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20, F to 71-80, F | BI1 to BI7 | Values for bi1, bi2, bi3, bi4, bi5. bi6, and bi7, 10 spaces for each. | See Eq. (III C3 a.2) for details. |
| 14: 17 | 6, I | IBI6 | Flag to vary bi6 | This line is present if and only if line 16 is also present. |
|  | 7, I | IBI7 | Flag to vary bi7 |  |
|  | 11-20, F to 71-80, F | dBI1 to dBI7 | Uncertainties on bi1, bi2, bi3, bi4, bi5. bi6, and bi7 |  |
| 14: 18 etc. Repeat Lines 15-17 for a total of 5 times at most. |  |  |  |  |
| 14: 19 | 1-5,A | WHAT | “BINS ” | Continuously varying channel width (bins per decade of energy). (Line may be omitted; line must be omitted if line 20 is included.) See Section III.C.3.a. |
|  | 6-10, I | Nbinpd | 1 < Nbinpd < 99999 |  |
| 14: 20 | 1-5, A | WHAT | “CHANN” | Channel crunch boundaries. (Line may be omitted. Line cannot be present if line 19 is present.) |
|  | 7, I | ICH | Flag to vary c | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20 | ECRNCH | Highest energy for this channel width c (eV) | See Section III.C.3.a. |
|  | 21-30 | CH | Values for c (ns) |  |
|  | 31-40 | dCH | Uncertainty for c (ns) |  |
| 14:21... Repeat line 20 as many times as needed. Lines 19 and 20 cannot be used together. |  |  |  |  |
| 14:Last (blank) |  |  |  |  |