| C:L | P,T | Variable name | Meaning (units) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 8:1 | 1-80, A | CROSS | Type of data, which must begin in first column on the line. Only those characters in CAPITAL letters are required. |  |  |
|  |  |  | TRANSmission |  | Use of transmission data rather than total cross section data is recommended; see Section XI.A for details. |
|  |  |  | TOTAL cross section |  |  |
|  |  |  | ELASTic or SCATTering |  |  |
|  |  |  | DIFFErential elastic |  | Elastic angular distribution |
|  |  |  | DIFFErentiAL REaction |  | Reaction angular distribution |
|  |  |  | REACTion , or INELAstic scattering, or FISSion |  | These three data types produce the same computation, unless further information is given. For details regarding the use of individual REACTion types, see Section II.B.1.c. See also card set 10.1:2 and 10.2:2, parameter IFEXCL. |
|  |  |  | FINAL state pairs = PAIRS in final state = |  | Using one of these two options is the preferred means of indicating the reaction data type. The equal sign must be followed by the names (from card set 4 or 4a) of the particle pairs to be included in the final state. |
|  |  |  | CAPTUre |  |  |
|  |  |  | SELF-indication |  | Section III.E.6 |
|  |  |  | ABSORption |  |  |
|  |  |  | ETA (“ETA” followed by 2 spaces) |  | Defined as |
|  |  |  | INTEGral |  | Section V.B |
|  |  |  | COMBInation |  | This option is no longer available. |
|  |  |  | CONSTant LINEAr QUADRatic DIRAC delta function 1/V |  | See Section V.E for details on these pseudo cross section types. |