Table X P.1. Variables appearing in the “suggel.inp” file

| Key word | Units | Description |
| --- | --- | --- |
| Parin |  | Input resonance parameter data file name. In the file, the resonance energy is given in eV and Γn (or gΓn) in meV. |
| Partyp |  | Type of neutron width in ‘parin= file. If partyp = 0, the parin file is in SAMMY.PAR file format and Γn=s are given; otherwise, the file is in rsap.gamma format and gΓn=s are given. Concerning the rsap.gamma file, see following this table. |
| Fout |  | Output list file name. |
| Parout |  | File name of the output resonance parameter file, which is in SAMMY.PAR format. |
| Awri |  | Atomic mass of target nuclide divided by the neutron mass. |
| Spin |  | Spin of the target nuclide (I). |
| Scattr | fermi | Scattering radius (R'). If scattr = 0.0, the program calculates it as R' = 0.123 (awri)1/3 +0.08 [10−12 cm] |
| Disp |  | Spin dispersion parameter (s). If disp = 0.0, the program calculates the value. Very large ‘disp= value (e.g., 999.) implies to use (2J+1) law, with which the level density is proportional to 2J+1. See further discussion following the table. |
| Bn | MeV | Neutron separation energy of (A+1) nuclide |
| Pair | MeV | Pairing energy |
| Denpar | MeV−1 | Level density parameter |
| Eini | eV | Lower energy boundary under the analysis |
| Efin | eV | Upper energy boundary under the analysis |
| Pcut |  | Probability cutoff. The suggested orbital angular momentum is adopted for a resonance for which the probability is higher than this value. If it is not the case, the l value is not altered from its original assignment as given in ‘parin= file. It is not effective with the input rsap.gamma file. |
| Optj |  | Method of J assignment. If optj = 0, the most probable J value is adopted; otherwise, the J value is assigned randomly. Regardless of this option, the original J value is maintained if the original l value is maintained due to the probability cutoff. See Section 2.6 of [SO01]. |
| Stn(i), i=1,2,3 | 10−4 | Neutron strength functions for s-, p-, and d-waves, respectively |
| Dzero | eV | Average level spacing of s-wave resonance. Level spacings of p- and d-waves are calculated in the program. |
| Frac(i), i=1,2,3 |  | A priori probability correction factors for s-, p-, and d-waves, respectively. See Section 2.2 of [SO01]. |
| Sam(i), i=1~max.18 |  | l value of spin group i in ‘parin’ file. This definition shall be kept in subsequent SAMMY run. The number of spin groups can be more than 18 in SAMMY run (e.g., in case of the analysis of a natural element), and it is also possible to define the spin group not to begin with 1 in SAMMY. However, the SUGGEL assumes that the spin group starts at 1. |
| Ajsam(i), i=1~max.18 |  | J value of spin group i in ‘parin’ file. This definition shall be kept in subsequent SAMMY run. |

Note from SAMMY author: This program does not run under the VMS operating systems.  In general, portability of SUGGEL is not guaranteed.