| C:L | P,T | Variable name | Meaning (units) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 10.2: 1 | Use this option for defining the spin groups only when one of the phrases “KEY-WORD PARTICLE-PAir definitions are given” or “PARTICLE-PAIR DEFINItions are given” occurs in card set 3, and when card set 4 or 4a is present. |  |  |  |  |
|  | 1-3, I | JJ | Spin group number (consecutive, beginning with 1) |  |  |
|  | 5, A | EXCL(JJ) | Flag to exclude this spin group from calculation |  | blank = include “X” = exclude |
|  | 8-10, I | NENT(JJ) | Number of entrance channels (must be > 0) |  | NENT + NEXT is the total number of particle channels for this spin group. |
|  | 13-15, I | NEXT(JJ) | Number of exit channels, excluding those which are also entrance channels (can be zero) |  |  |
|  | 16-20, F | SPINJ(JJ) | Integer or half-integer spin for resonances in group J |  | Positive for even parity and negative for odd |
|  | 21-30, F | ABNDNC (JJ) | Isotopic abundance for this spin group |  | (May be overwritten in PARameter file) |
| 10.2: 2 | 3-5, I | N | Channel number |  |  |
|  | 6-7 | (blank) |  |  |  |
|  | 8-15, A | PPTYPE | Name of particle pair |  | Must be identical to one of the particle-pair names given in card set 4 or 4a |
|  | 18, I or A | IFEXCL (N,JJ) | 1 if exclude this channel from final-state calculation for reaction cross section; 0 or blank if include. X (or x) to exclude this channel from inclusion anywhere in the calculation. |  | This flag defines the particular reaction to be calculated. (See card set 8 above.) This option (“X”) should be used only for purposes of testing your input; it should not be used for regular production runs. See test case tr152 for examples using this feature. |
|  | 19-20, I | LSPIN (N,JJ) | Orbital angular momentum for channel |  | Non-negative integer |
| 10.2: 2 cont. | 21-30, F | CHSPIN (N,JJ) | Channel spin (integer or half-integer, sign implies parity) |  | Channel spin is the vector sum of the spins of the two particles in the channel. |
|  | 31-40, F | ENBND (N,JJ) | Boundary condition (eV) Used only if ISHIFT = 1 or SHift = Yes. Default: ENBND = - LSPIN |  | If ENBND > 0, then B = Sl(ρ), where ρ = ka and is the center-of-mass momentum for energy ENBND (Section II). If ENBND < 0, then B = ENBND. |
|  | 41-50, F | RDEFF (N,JJ) | Effective radius (fermi) |  | Needed here only if different from the value of the effective radius given in the particle-pair definitions. Is ignored if Radius parameters are given in the PARameter file. |
|  | 51-60, F | RDTRU (N,JJ) | True radius (fermi) |  | Needed here only if different from the value of the true radius given in the particle-pair definitions. Is ignored if Radius parameters are given in the PARameter file. |
| 10.2: 3,4,etc. Repeat Line 2 for a total of (NENT + NEXT) lines |  |  |  |  |  |
| 10.2: 5,6,etc. Repeat Lines 1,2,3,4,etc, once for each spin group |  |  |  |  |  |
| 10.2: Last (blank) |  |  |  |  |  |