Table VI B.2. Format of the PARameter file

Comment 1:	The two numbers in the first column (labeled “C:L”) represent the card set number (before the colon) and the line number within that card set (after the colon).

Comment 2:	Numbers in the second column (labeled “P, T”) indicate position number within the line.  Letters indicate format type:  integer format (“I”, which implies a number without a decimal point and requires that the number be in the right-most columns), real format (“F”, which requires a number with a decimal point), or alphanumeric (“A”, which can be numbers or letters or symbols).

| C:L | P, T | Variable Name | Meaning (units) | Notes |
| --- | --- | --- | --- | --- |
| 1:1 | 1-11, F | Eλ | Resonance energy Eλ (eV) |  |
|  | 12-22, F | Γγ | Capture width (milli-eV) |  |
|  | 23-33, F | Γc1 | Particle width for channel 1 (milli-eV) | If any particle width Γ is negative, SAMMY uses for the width and set the associated amplitude γ to be negative. That is, SAMMY uses, where α is the appropriate factor (see Section II.A.1). |
|  | 34-44, F | Γc2 | Particle width for channel 2 (milli-eV) |  |
|  | 45-55, F | Γc3 | Particle width for channel 3 (milli-eV) |  |
|  | 56-57, I | ISE | Vary Eλ? | 0 = no, 1 = yes, 3 = PUP (See Section IV for meanings of “vary” and “PUP”.) |
|  | 58-59, I | ISγ | Vary Γγ? | 0 = no, 1 = yes, 3 = PUP |
|  | 60-61, I | ISc1 | Vary Γc1? | 0 = no, 1 = yes, 3 = PUP |
|  | 62-63, I | ISc2 | Vary Γc2? | 0 = no, 1 = yes, 3 = PUP |
|  | 64-65, I | ISc3 | Vary Γc3? | 0 = no, 1 = yes, 3 = PUP |
|  | 66-67, I | IGROUP | Quantum numbers for this resonance are those of group number IGROUP (card set 10.1 or 10.2 in Table VIA.1) | If IGROUP is negative or greater than 50, this resonance will be omitted from the calculation. |
|  |  |  | (To use more than 50 spin groups, include the phrase “USE I4 FORMAT TO REAd spin group number” in the INPut file card set 3. Then use Columns 66-68 for IGROUP, and columns 70-82 for X. The maximum number of spin groups is then 500; values greater than 500 indicate omitted resonances.) |  |