| C:L | P,T | Variable name | Meaning (units) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 6:1 | 1-10, F | DELTAB | Minimum channel width for interval from EMIN to EMAX (μs) |  | This card set is included only if DELTAG< 0 in card set 5. |
|  | 11-15, I | NCF | Number of crunch boundaries (energy intervals) |  |  |
| 6:2 | 1-10, F 11-20, F etc. | (BCF(I), CF(I), I=1,NCF) | BCF(I) = maximum energy for this crunch factor (read in order of increasing energy) (eV) CF(I) = crunch factors (integer units of DELTAB) |  | The value of Δtc to be used in the energy range BCF(I-1) < E < BCF(I) is CF(I)×DELTAB. See Eq. (IIIC1a.7) and the following. |
| 7:1 | 1-10, F | CRFN | Channel radius (fermi) If CRFN is input as zero, its value is computed as 1.45×(1.008665 +AW)1/3. |  | Value may be overwritten in card set 4 or 7 of the PARameter file (see Table VIB.2). |
|  | 11-20, F | THICK | Sample thickness (atoms/barn). |  | Needed only for TRANSmission or for TOTAL cross section or for multiple-scattering corrections. |
|  | 21-30, F | DCOVA or ERRXC2 | DCOVA = Constant term in data covariance (default = 0.0) |  | Data covariance matrix has additional term of the form (DCOVA + Ei × DCOVB) × (DCOVA+ EJ×DCOVB). Usually these terms will be set to zero. |
|  | 31-40, F | DCOVB or SIGMIN | DCOVB = Linear term in data covariance (default = 0.0) For a description of ERRXC2 and SIGMIN, see Section V.A on reconstructing point-wise cross sections. |  |  |
|  | 41-50, F | VMIN | Minimum absolute uncertainty (standard deviation) on experimental data Often set to zero |  | If an experimental uncertainty in the DATa file is smaller than VMIN, SAMMY will increase the value to VMIN. |