Table VI E.2. Complete list of input files for SAMMY runs, in the order in which they will occur.  Only a subset of the SAMMY prompts will occur for any given SAMMY run.

|  | SAMMY prompt | What is needed |  | Invoking command |
| --- | --- | --- | --- | --- |
| 1. | What is the name of the INPut file? | INPut file name plus extension. Maximum number of characters is 70. |  |  |
| 2. | What is the name of the PARameter file? | PARameter file name. Maximum number of characters is 70. |  |  |
| 3. | What is the first data file name? EMIN? EMAX? | DATa file name followed by the minimum and maximum energies (in EV) for this step in F format, separated by commas or spaces. |  |  |
| 4. | What is name of input file for ENDF output? | The name of the “ndf” file, described in Section VI.F.2. |  | ENDF/B-VI FILE 2 IS wanted, or ENDF |
| 5. | What are names of WY files? | Names of the files produced by “yw” runs (described in Section IV.E.1) in preparation for simultaneous fitting to be accomplished in this run. Each file is specified on a separate input line, ending with a blank line. |  | GENERATE Y AND W MATrices or WY |
| 6. | What is name of file with values of direct capture? | See Section II.B.4 for details. |  | ADD DIRECT CAPTURE Component |
| 7. | What is name of parameter COVariance file? | See Section VII.B and Table VII B.1 for details. |  |  |
| 8. | What is name of file for edge effects? | This SAMMY-produced file contains geometric information for single-scattering corrections. See Section III.D for more information. |  | FILE WITH EDGE EFFECts already exists |
| 9. | What is name of implicit data covariance file? | Section VI.C.3.c has details. |  | IMPLICIT DATA COVARIance matrix is to be used |
| 10. | What is name of AVG file? | See Section VI.F.1 for details. | AVERAGE OVER ENERGY ranges, GROUP AVERAGE OVER Energy, BONDARENKO AVERAGE Over, UNWEIGHTED ENERGY Average, or ENERGY-AVERAGE USINg constant flux |  |
| 11. | What is name of Maxwellian average file? | Section VI.F.4 has details. |  | MAXWELLIAN AVERAGED capture cross sections are wanted or STELLAR AVERAGED CAPture cross sections are wanted |
| 12. | What is name of endf/b file 3? | See Table VI F4.1. |  | ADD CROSS SECTIONS From endf/b file 3 |
| 13. | What is name of Crystal-Lattice Model file? | See Section VI.F.5. |  | USE CRYSTAL LATTICE model of doppler broadening, or CLM |
| 14. | What is name of integral data file? | See Sections V.B and VI.D for details. |  |  |
| 15. | What is name of file with information for File 3? | Contains information needed for creating ENDF File 3 for the unresolved resonance region. (Similar to *.ndf file, # 4 above.) See Section VI.F.2. |  | PUT COVARIANCE MATRIx into endf file 32 |
| 16. | What is name of file with PUP covariances? | See end of Section IV.D.2 for further information. |  | PUP COVARIANCE IS IN an ascii file |
| 17. | What is name of file with E-dependent initial uncertainty multiplier? | See Section IV.E.6 for details. |  | E-DEPENDENT INITIAL uncertainty multiplier |
| 18. | What is name of file containing new parameter uncertainties? | See Section IV.E.2 for details. |  | P COVARIANCE MATRIX is correct, u is not and MODIFY P COVARIANCE matrix before using |
| 19. | What is name of file containing Y2 values? | See Section III.D and [NL08] for details. |  | Y2 VALUES ARE TABULated |
| 99. | What is new EMIN? EMAX? DATa file name? | Energy range (in eV) for the next step, in F format, separated by commas or spaces. New data file name, when needed. End in blank line. With the large capacity of modern computers, this option is seldom used anymore. |  |  |