| C:L | P, T | Variable Name | Meaning (units) | Notes |
| --- | --- | --- | --- | --- |
| 1a:1 | When this format is used for input of resonance parameters, the command “REDUCED WIDTH AMPLITudes are used for input” must be included in the INPut file. |  |  |  |
|  | 1-11, F |  | Square root of the resonance energy | When , the quantity to be given here is . |
|  | 12-22, F |  | Reduced-width amplitude for the eliminated capture channel |  |
|  | 23-33, F |  | Reduced-width amplitude for channel 1 |  |
|  | 34-44, F |  | Reduced-width amplitude for channel 2 |  |
|  | 45-55, F |  | Reduced-width amplitude for channel 3 |  |
|  | 56-57, I | ISE | Vary ? | 0 = no, 1 = yes, 3 = PUP (See Section IV for meanings of “vary” and “PUP”.) |
|  | 58-59, I | ISγ | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 60-61, I | ISc1 | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 62-63, I | ISc2 | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 64-65, I | ISc3 | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 66-67, I | IGROUP | Quantum numbers for this resonance are those of group number IGROUP (card set 10.1 or 10.2 in Table VIA.1) | If IGROUP is negative or greater than 50, this resonance will be omitted from the calculation. |
|  |  |  | (To use more than 50 spin groups, include the phrase “USE I4 FORMAT TO REAd spin group number” in the INPut file card set 3. Then use Columns 66-68 for IGROUP, and columns 70-82 for X. The maximum number of spin groups is then 500; values greater than 500 indicate omitted resonances.) |  |
| 1a:1 cont. | 68-80, F | X | For many cases, this variable is not used (columns may be left blank). However, if X < 0, then values of γ for other particle channels are included on the following line. For other possible meanings for X, see card set 1. |  |
| 1a:2 | 1-11, F |  | Reduced-width amplitude for channel 4 | This line is present only if X < 0 on the preceding line |
|  | 12-22, F |  | Reduced-width amplitude for channel 5 |  |
|  | 23-33, F |  | Reduced-width amplitude for channel 6 |  |
|  | 34-44, F |  | Reduced-width amplitude for channel 7 |  |
|  | 45-55, F |  | Reduced-width amplitude for channel 8 |  |
|  | 56-57, I | ISc4 | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 58-59, I | ISc5 | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 60-61, I | ISc6 | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 62-63, I | ISc7 | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 64-65, I | ISc8 | Vary ? | 0 = no, 1 = yes, 3 = PUP |
|  | 66-67, I | IGROUP | must be same as previous line |  |
|  | 68-80, F | X | As above, negative to indicate continuation line. Positive for uncertainty on resonance energy, or for off-diagonal DCM. |  |
| 1a:3,4, etc Repeat line 2 as needed, to include all non-zero reduced-width amplitudes. |  |  |  |  |
| 1a:5 etc, Repeat lines 1 – 4 as many times as needed, until all resonances are specified. |  |  |  |  |
| 1a:Last End with a blank line. |  |  |  |  |