| C:L | P, T | Variable Name | Meaning (units) | Notes |
| --- | --- | --- | --- | --- |
| 16:1 | 1-80, A | WHAT | “USER-Defined resolution function” | See Section III.C.5. This card set could be put into the INPut file rather than PARameter file if no values are to be varied. |
| 16:2 | 1-5, A | WHAT | “BURST” | Line is optional. |
|  | 7, I | IFBRST | Flag to indicate whether burst width is to be varied | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20, F | BURST | Value of square burst width (ns) |  |
|  | 21-30, F | dBURST | Uncertainty on burst (ns) |  |
| 16:3 | 1-5, A | WHAT | “CHANN” | Channel crunch boundaries. (Line is optional.) |
|  | 7, I | ICH | Flag to vary c | 0, 1, or 3 to keep, vary, or PUP. |
|  | 11-20 | ECRNCH | Highest energy for this channel width c (eV) | See Section III.C.5. |
|  | 21-30 | CH | Values for c (ns) |  |
|  | 31-40 | dCH | Uncertainty for c (ns) |  |
| 16:4,5... Repeat Line 3 as many times as needed |  |  |  |  |
| 16:6 | 1-5, A | WHAT | “FILE=” |  |
|  | 6-75, A | NAME | Name of file containing user-defined resolution function |  |
| 16:7,8... Repeat Line 6 as many times as needed |  |  |  |  |
| 16:Last (blank) |  |  |  |  |