| Category | D | Statements | Notes | # |
| --- | --- | --- | --- | --- |
| Multiple-scattering corrections | D | DO NOT INCLUDE SELF- shielding or multiple-scattering corrections, or NO SELF-SHIELDING ANd multiple-scattering corrections | Calculate the cross section but do not include any self-shielding or multiple-scattering corrections. | 251 252 |
|  |  | USE SELF SHIELDING Only, no scattering, or SELF SHIELD, or INCLUDE ONLY SELF SHielding and not Multiple scattering | Only self-shielding is to be included; no single- or double-scattering (see Section III.D). | 253 254 255 |
|  |  | USE SINGLE SCATTERINg plus self shielding, or SINGLE | Include self-shielding plus single-scattering corrections. | 256 257 |
|  |  | INCLUDE DOUBLE SCATTering corrections, or USE MULTIPLE SCATTERing plus single scattering, or DOUBLE, or MULTIPLE | Include self-shielding, single-scattering, and double-scattering corrections (see Section III.D). | 258 259 261 260 |
|  |  | INFINITE SLAB, or NO FINITE-SIZE CORREctions to single scattering | Single-scattering correction does not include edge effects. | 263 262 |
|  | D | FINITE SLAB, or FINITE SIZE CORRECTIons to single scattering | Edge effects corrections are included for single scattering. | 265 264 |
| Multiple-scattering corrections (cont.) | D | MAKE NEW FILE WITH Edge effects | See Section III.D. This statement is ignored if “INFINITE SLAB” is specified. Be sure to rename and save the SAMMY.SSM file if you wish to reuse it. | 266 |
|  |  | FILE WITH EDGE EFFECts already exists | Use this option to conserve run time. Include the name of this file (which was generated by a previous SAMMY run) after the name of your DATa file. | 267 |
|  |  | MAKE PLOT FILE OF MUltiple scattering pieces | Store Y0, Y1, and Y2 in file SAM53.DAT, to be read by program SAMCPR. | 268 269 |
|  |  | NORMALIZE AS CROSS Section rather than yield, or CROSS SECTION | See Section III.D. Note than normalization must be specified explicitly. SAMMY will abort if the INPut file does not specify which normalization is to be used. | 270 271 |
|  |  | NORMALIZE AS YIELD Rather than cross section, or YIELD |  | 272 273 |
|  |  | NORMALIZE AS (1-E)SIgma |  | 274 |
|  |  | PRINT MULTIPLE SCATTering corrections | Create a table of E, Y0, Y1, Y2, and Y (energy plus components of the corrected yield) in file SAM012.DAT. | 275 |
|  |  | PREPARE INPUT FOR MOnte carlo simulation, or MONTE CARLO | This is the initial SAMMY run to generate cross sections for use in Monte Carlo simulation of multiple-scattering effects. | 276 277 |
|  |  | Y2 VALUES ARE TABULAted | When SAMMY’s calculation of double-plus scattering is inadequate, tabulated values for Y2, can be used. See Section III.D for information on creating an using tabulated values. | 247 |
| Multiple-scattering corrections (cont.) | D | USE QUADRATIC INTERPoltion for y1 | Quadratic interpolation is used to generate the Q-functions at specific values of the arguments | 250 |
|  |  | USE LINEAR INTERPOLAtion for y1 | Linear interpolation is used to generate the Q-functions at specific values of the arguments. This is neither as accurate nor as efficient as quadratic interpolation, but may be faster under certain conditions. | 249 |
|  |  | DO NOT CALCULATE Y0 | Used for debugging purposes only | 248 |