Table VIII B.1.  Formats for original PARameter file for treatment of the

unresolved resonance region

| Card set | Line No. | Description |
| --- | --- | --- |
| 1 | 1–4 | First four lines are alphanumeric title |
| 2 | 1 | Number of iterations, fitting tolerance (essentially delta chi squared). Note that integers are to be specified as real numbers. All formats are F10. |
|  | 2 | Mass in amu, radius in Fermi (or use default), neutron binding energy in MeV, pairing energy PE in MeV. Again, formats are F10; note that the energy units are MeV, as opposed to the usual SAMMY standard of eV. |
| 3 | 1,2,... | Center-of-mass excitation energy (in eV), spin, and parity for the nth target level (beginning with ground state). Repeat as many times as needed. |
|  | Last | (Blank) |
| 4 | 1 | Strength function Sc, uncertainty, distant-level parameter, uncertainty, radiation width in eV, uncertainty, mean level spacing D in eV for l = 0 |
|  | 2 | Strength function, uncertainty, distant-level parameter, uncertainty, radiation width in eV, uncertainty, for l = 1 |
|  | 3 | Strength function, uncertainty, distant-level parameter, uncertainty, radiation width in eV, uncertainty, for l = 2 |
|  | 4,5,... | As above, for higher l values as needed |
|  | Last | (Blank) |
| 5 | 1 | For the lowest J value for l = 0, Average fission width (eV) Degree of freedom for fission width distribution Hill-Wheeler threshold energy Hill-Wheeler threshold width Uncertainty on the average fission width |
|  | 2,3,… | Repeat line 1 for each possible value of J for l = 0. |
| 5, cont. | 4,5,… | Repeat lines 1-3 for each possible value of J for l = 1, 2, ... For a given spin J and parity (even or odd l), only one set of values is actually used for and the other parameters. Nevertheless, all J and l must be included in this list. Only the values associated with the lowest l value will be used for the calculations; the other values will be ignored. |
|  |  | For example, the ground state of 235U is 7/2−. For l = 0, J π = 3−, 4−. For l = 1, J π = 2+, 3+, 4+, 5+. For l = 2, J π = 1−, 2−, 3−, 4−, 5−, 6−. The 3−and 4− values used in the calculations will be those given for l = 0. The values given for J π = 3−and 4− with l = 2 will be ignored. |
|  | Last | (Blank) |
| 6 | 1 | Type of cross-section data (TOTAl, CAPTure, FISSion, or INELastic) |
|  | 2 | Uncertainties are RELAtive or ABSOlute |
|  | 3 | Energy, cross section, and uncertainty for first data point. Normalization and uncertainty (a and Δa) for this data set. |
|  | 4,... | Energy, cross section, uncertainty. (Note: if RELAtive then these need to be specified only for first data point; the others are assumed to be the same.) |
|  | Last | (Blank) |
| 6x | All | Repeat card set 6 as many times as needed, in any order |
| 7 | 1 | The single word “NORMALIZATION”. (Card set 7 is present only if the command “EXPERIMENTAL DATA ARe in separate files” appears in card set 3 of the INPut file.) |
|  | 2 | Type of cross section, normalization parameters a, Δa, b, Δb, c, Δc, where the normalization for this data set is given by |
|  | 3,etc. | Repeat Line 2 once for each data set. Normalizations must appear in the same order in which the data sets appear. SAMMY will check to be sure the data types are consistent. |