| Category | D | Statements | Notes | # |
| --- | --- | --- | --- | --- |
| Cross section calculation details |  | USE POLAR COORDINATEs for fission widths | When there are two channels for fission, the pair may be treated as a vector, in which case the two independent variables are the magnitude of the vector and the polar angle, rather than the two widths. Compare, for example, with the Vogt formalism [EV58]. | 191 |
|  |  | NUMERICAL DERIVATIVEs for resonance parameters | Debug command, to be used only for testing that analytic derivatives are correct. | 193 |
|  | D | DO NOT USE S-WAVE CUtoff | See Section II.D.1.b for a description of these options. | 194 |
|  |  | USE S-WAVE CUTOFF |  | 195 |
|  |  | USE NO CUTOFF FOR DErivatives or cross sections |  | 186 |
|  |  | USE ALTERNATIVE COULomb functions | Use the more-accurate (but also more time-consuming) version of the Coulomb penetrabilities and shift factors. See Section II.C.4 for details. | 196 |
|  |  | ADD DIRECT CAPTURE Component to cross section | See Section II.B.4 for details. | 197 |
|  | D | LAB NON COULOMB EXCItation energies | The excitation energy (for channels for which there is no Coulomb interaction) is defined in the laboratory rather than the center-of-mass system. See parameter ECHAN in card set 10.1 or 10.2 of the INPut file, Table VIA.1. | 67 |
|  |  | CM NON COULOMB EXCITation energies | The non-Coulomb excitation energy is given in the cm system rather than the laboratory system. | 68 |
|  | D | LAB COULOMB EXCITATIon energies | For Coulomb channels, the excitation energy ECHAN (card set 4 or 4a or 10.1 of the INPut file) is in the laboratory system. | 69 |
|  |  | CM COULOMB EXCITATIOn energies | For Coulomb channels, the excitation energy is in the center-of-mass system. | 70 |
| Cross section calculation details, continued |  | ADD ELIMINATED CAPTUre channel to final state | When some capture channels are treated individually and others in aggregate, this command will include both in the calculation of capture cross section. See Section II.B.2 for details. | 10 |