III.E.8.  Energy-Scale Dependence on t0 and L

When analyzing two or more independent data sets, the problem of displacement between energy scales often arises.  Proper alignment of energy scales has traditionally been a task for the analyst; however, help is available from SAMMY.  Two methods are described here.

First method:

In time-of-flight measurements, the energy of the incident neutron is determined from the flight-path length L, the flight time t, and the initial time t0 as

By treating L and t0 as variables (parameters) and searching for those values of L and t0 which provide the best fit of experimental measurement to the calculated cross section, it is possible to adjust the energy scales to provide agreement between different data sets.  See card set 11, line 6, of the PARameter file, Section VI, for input details.

Derivatives of the cross section with respect to flight-path length L and initial time t0 are given in terms of the derivative with respect to energy as

and

Recall that the cross section σ is given in terms of the scattering matrix U.  The energy-dependence of σ is therefore that of U (via ρ) and of the  multiplicative factor.  That is, this derivative is given by

which can be rewritten as

Combining this equation with Eqs. and  gives

and

The derivative of the cross section with respect to ρ is again, as in Section II.D.1.c, found by substituting Eq. (II D1 c.9) into Eq. (II D1 c.6).

Note that relativistic corrections are often added to the expression in Eq. ; such corrections are ignored in SAMMY's treatment of the t0 dependence.

Within SAMMY the value of L is not varied directly; rather, L is set equal to the product of the “initial” value Li times a dimensionless quantity L0.  This dimensionless multiplier is generally given an initial value of 1.0, and the value is varied in the SAMMY run.

In preliminary studies, what has been found practical is to use this option somewhat apart from the mainstream of SAMMY analyses.  The procedure that worked best in our trials was to first obtain a reasonably good fit to the “best” data set (the data set which, in the analyst’s judgment, has the “most correct” energy scale), with particular emphasis on obtaining accurate resonance energies.  Next, modify the parameter file to vary only L0 and t0 (i.e., do not vary resonance energies or other resonance parameters) and make a SAMMY run to determine the best values for L0 and t0 for the data set in which the energy scale is “wrong.”  Once those values have been determined, generate a modified data set which has the corrected energy scale and use the modified set in subsequent work.  The code SAMFTZ, described in Section X.G of this manual, can be used to generate the modified data set.

Second method:

Begin by fitting each data set separately from the others, establishing in particular appropriate energies for the largest resonances.  Choose one data set (probably the transmission measurement with the longest flight path) as the standard.  For each other data set, compare energies for individual resonances to those in the standard data set.  By considering the relative energy difference as a function of resonance energy, a fit can be made to the functional form

in which E is the energy of the resonance in the standard data set, and  the energy in the other data set.  Equation  is the appropriate functional form, as can be seen by taking increments of Eq. ,

or

Once appropriate values are determined for a and b, all energies in the non-standard data set can be modified by replacing the original (incorrect) energy  by , where

Solution of this equation for  can be done iteratively, with as starting value.