| C:L | P, T | Variable Name | Meaning (units) | Notes |
| --- | --- | --- | --- | --- |
| 4:1 | 1-80, A | WHAT | “BROADening parameters may be varied” | Values for parameters in this card set may be specified in the INPut file if they are not to be varied (card sets 5 and 7 of Table VIA.1). Alternatively, this entire card set may be inserted at the end of the INPut file when parameters are treated as PUPs. |
| 4:2 | 1-10, F | CRFN | Matching radius (F) | In many cases this value is overruled (e.g., card set 7 or 7a). |
|  | 11-20, F | TEMP | Effective temperature of the sample (K) |  |
|  | 21-30, F | THICK | Sample thickness (atoms/barn) |  |
|  | 31-40, F | DELTAL | Spread in flight-path length (m) | See Section III.C.1.a for a detailed description of these parameters. |
|  | 41-50, F | DELTAG | The absolute value of DELTAG is the full width at half max of a Gaussian resolution function whose variance is equivalent to that of the square pulse (μs). See Section III.C.1.a for details. | In card set 5 of the INPut file, a negative value of DELTAG is used as a marker to indicate that channel widths and crunch boundaries are included in card set 6. Here in card set 4 of the PAR file, DELTAG can be either negative or positive, but only ¦DELTAG¦ has any meaning. It is acceptable to have a negative value of DELTAG in the INPut file, so that crunch boundaries and channel widths will be read, and then have a completely different value of DELTAG in the PARameter file. SAMMY computations will make use of (1) the crunch boundaries and channel widths from the INPut file and (2) the absolute value of DELTAG from the PARameter file. See test case tr048 for examples. |
| 4:2 cont. | 51-60, F | DELTAE | e-folding width of exponential resolution function (μs) | See Section III.C.1.b for details. |
|  | 61-62, I | Icrfn | Flag to vary this parameter; values -2, 0, and 1 may be used, but -1 and 3 may not (see below for meanings of these values). |  |
|  | 63-64, I | Itemp | Flags to vary these parameter. Meanings are as follows: -2 → do not vary this parameter. Use the value given in card sets 5, 6, or 7 of the INPut file rather than this value. -1 → do not vary this parameter. Use the value given here in the PARameter file rather than the value given in the COVariance file (if it exists). (Note that only those parameters that were not varied in earlier runs can be changed in this manner.) 0 → do not vary this parameter. Use the value given in the COVariance file if it exists; otherwise, use this value. 1 → do vary this parameter. For the starting value, use the value given in the COVariance file if it exists. 3 → PUP this parameter. Use the value given here. (See Section IV.D.2 for a description of propagated uncertainty parameters.) |  |
|  | 65-66, I | Ithick |  |  |
|  | 67-68, I | Ideltal |  |  |
|  | 69-70, I | Ideltag |  |  |
|  | 71-72, I | Ideltae |  |  |
| 4:3 | 1-10, F 11-20, F … 51-60, F | dCRFN dTEMP … dDE | Uncertainty on CRFN Uncertainty on TEMP … Uncertainty on DELTAE | This line is optional, unless additional parameters are given on subsequent lines. If this line is absent, prior uncertainties are set to FUDGE value of parameter. |
| 4:4 | 1-10 | DELTC1 | Width of Gaussian, constant in energy (eV) | See Section III.C.1.a for details. |
|  | 2-20 | DELTC2 | Width of Gaussian, linear in energy (unitless) |  |
|  | 61-62,I | Ideltc1 | Flag to vary DELTC1 |  |
|  | 63-64,I | Ideltc2 | Flag to vary DELTC2 |  |
| 4:5 | 1-10 | dDC1 | Uncertainty on DELTC1 |  |
|  | 11-20 | dDC2 | Uncertainty on DELTC2 |  |
| 4:4 | (blank) |  |  |  |