| Category | D | Statements | Notes | # |
| --- | --- | --- | --- | --- |
| URR controls |  | EXPERIMENTAL DATA ARe in separate files | See Section VIII.B for details of input to unresolved resonance region (URR) calculations. | 371 |
|  |  | NO ANNOTATED PARAMETer file for urr input | Input for URR is essentially the original format as in the FITACS code; see Table VIII.B.1. | 372 |
|  | D | ANNOTATED PARAMETER file for urr, or ANNOTATED | Input for URR is “annotated file”, which contains key words and phrases to make the input more human-legible. See Section VIII.B and Table VIII.B.2 for details. | 373 374 |
|  | D | OUTPUT IN ANNOTATED parameter file for urr | Output parameter file is annotated. (This is always true; SAMMY now contains no option for writing non-annotated output, although that can still be used as input). | 375 |
|  |  | ENDF/B-VI FILE 2 IS wanted, or ENDF | Resonance parameters are to be written into the format required for ENDF/B-VI File 2. See Section VI.F.2 for details. | 291 292 |
|  | D | USE ALL EXPERIMENTAL data points, or DO NOT USE ENERGY LImits as given in the input file | All data points appearing in the initial URR PARameter file (see Section VIII.B) are to be fitted during the analysis. Energy limits given in the INPut file will be ignored. | 376 377 |
|  |  | USE ENERGY LIMITS AS Given in the input file | The energy range specified in card set 2 of the INPut file will be used to define which data points are to be included in the analysis. | 378 |
|  |  | INCLUDE MIN & MAX ENergies in endf file | When creating ENDF File 2 for LRU = 2, include Emin and Emax from the SAMMY run as part of the energy list. Examples are in test cases tr127, tr128, tr133, and tr134. | 379 |
| URR controls (cont.) |  | CALCULATE WIDTH FLUCtuation factors more accurately, or CALCULATE DRESNER INtegrals more accurately | Use 1001 grid points rather than the default 101 points for calculating the Dresner integrals. See tr073 for examples. | 380 370 |
|  |  | MOLDAUER PRESCRIPTIOn is to be used, or MOLDAUER | Moldauer’s “effective degree of freedom” is to be used, to compensate for strong overlap of resonances. See page 352 of FF00. | 322 323 |