III.E.3.b.   User-supplied data-reduction parameters

When uncertainties inherent in the parameters of the data-reduction process are carefully propagated through that process, the covariance matrix associated with the reduced data is off-diagonal. While SAMMY can handle off-diagonal data covariance matrices, it is often more convenient as well as more informative to treat the data covariance matrix as diagonal and incorporate the uncertainty information directly into the solution of Bayes' equations.

To understand how this is done, let d represent the raw (unreduced) data and D the reduced data.  The relationship between D and d is

where f i is some function of {d} (which represents the entire set of raw data) and of {q} (which represents the set of data-reduction parameters).  A small increment in D may then be written as

Squaring this expression, taking expectation values, and noting that the and are mutually independent give

for the covariance between  and .  In Eq. , use has been made of the uncorrelated nature of the raw data:

The first term in Eq.  represents the statistical uncertainty on D i .   Generally, it is diagonal with respect to i and i', since  f i  will usually depend only on d i and not on other di', but we have included off-diagonal terms for the sake of completeness.  The second term contains information from the data-reduction process.  Generally, this term is off-diagonal with respect to i and i'; that is, the reduced data D i all depend upon the same parameters qk, and thus are correlated.  Thus the data covariance matrix Vi i=  to be used in Bayes' equations involves off-diagonal terms.

It is possible for the full covariance matrix to be calculated from Eq.  and read directly into SAMMY (see Section VI.C.2).  However, all data points that are connected by off-diagonal terms need to be included in the same analysis.  Because typical time-of-flight  experiments may involve thousands of data points, this process is often impractical computationally.

An alternative method of handling uncertainties on data-reduction parameters is to treat those parameters as variables whose value is to be determined by the analysis process, that is, to treat them in the same fashion as the resonance parameters.

In the development of Bayes’ equations (Section IV), the conditional probability p(D|PX) was needed.  Since the data-reduction parameters {q} are now considered to be a part of the set {P}, Eq. (IV A1.3) requires the probability of D being true given that {q} are exactly the correct values.  That is, we use the data covariance matrix V formed from only the first term in Eq. , the second term giving zero if the values for{q} are exact.

Since data D, as well as theory T, now depend on parameters P, Eq. (II A1.4) must be expanded to include

in which we have set the partial derivative of Di  with respect to qk  equal to gik; that is,

Therefore, by expanding the set of parameters {P} to include the data-reduction parameters {q}, and by redefining Gik, the partial derivative of the theory Ti  with respect to the parameter Pk , to be the partial derivative of (Ti − Di ) with respect to P k , the equations take on the exact form discussed in Section IV.A.

To use this method in SAMMY, the analyst must generate the partial derivatives gik prior to the SAMMY runs, and provide the values to SAMMY.  (The code ALEX [DL83 and NL84] may be used for this purpose.)  Input details are given in Section VI.C of this report.

Caveat:  This option has been available in SAMMY almost from the beginning, but it has (to the author’s knowledge) never been used extensively and hence never been thoroughly tested.  Moreover, the techniques used are not optimal.  Unless there is demand for this option, it will probably be discontinued with the next release of the code.