| C:L | P,T | Variable name | Meaning (units) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 11: 2 | This line is optional |  |  |  |  |
|  | 1-10, F | XTMIN | Lowest value of log of total cross section to be used in evaluating Q for single-scattering correction (barns). See Section III.D for details. |  | Default = -20.0 |
|  | 11-20, F | XTMAX | Largest value of log of total cross section to be used in evaluating Q |  | Default = 20.0 |
|  | 21-50 |  |  |  | Currently unused |
|  | 51-55, I | JTHETA | Number of points in theta integration near cos(theta) = 1 |  | Default = 5 |
|  | 56-60, I | KTHETA | Number of points in theta integration near cos(theta) = 0 |  | Default = 5 |