| C:L | P, T | Variable Name |  | Meaning (units) | Notes |
| --- | --- | --- | --- | --- | --- |
| 9:8a | 1-5, A | WHAT |  | “NE110” | NE110 detector is used. |
|  | 7, I | IFDEL |  | Flag for δ | (Note that you cannot use both LITHI and NE110). |
|  | 8-10, I | NMDETS |  | Number of cross-section values and energies to be included (i.e., number of line # 9's to be read) | If zero, default values are used; default material is CH1.104. |
|  | 11-20, F | DELTA |  | δ (mm) |  |
|  | 21-30, F | dDELTA |  | Uncertainty on δ (mm) |  |
|  | 31-40, F | DENSTY |  | λ (number of molecules per mm.b of detector) | Default is 0.0047 |
| 9:9a | 11-20, F | ENDETS(i) |  | Maximum energy for this value of SIGDTS (eV) | These lines occur only with the NE110 detector, and only when NMDETS > 0. Repeat this line a total of NMDETS times. |
|  | 21-30, F | SIGDTS(i) |  | Total cross section for detector material (barns) |  |
| 9.10 | 1-5, A | WHAT |  | “CHANN” | Channel widths and crunch boundaries are given here. |
|  | 7, I | Ifchnn |  | Flag if CHANN is varied | 0 if fixed, 1 if vary, 2 if PUP |
|  | 11-20, F | ECRNCH(i) |  | Maximum energy for this channel width (eV) |  |
|  | 21-30, F | CHANN(i) |  | Channel width c (nsec) |  |
|  | 31-40, F | DCHANN(i) |  | Uncertainty on channel width c (nsec) |  |
| 9:11 | Repeat this line as many times as needed |  |  |  |  |
| 9:12 | (blank) |  |  |  |  |