| C:L | P,T | Variable name | Meaning (units) |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4a:1 | 1- 8, A | NAMPP | Alphanumeric name for this particle pair |  | This card set is present only when the command “PARTICLE-PAIR DEFINItions are used” occurs in card set 3 above. |  |
|  | 9-10, A | Aa | Type of particle for first particle in pair (incident particle, if appropriate) |  | For special particle types, charge, mass, spin, and parity do not need to be given in the INPut file. These types include “ n”, “ p”, “de”, “he”, “h3", “tr”, and “al”. |  |
|  | 11-12, A | Ab | Type of particle for second particle in pair |  | ditto |  |
|  | 13-15, I | Za | Charge number for first particle |  | (not needed if one of the special particle types is used, or if the other particle is a neutron, or if this is a fission pair) |  |
|  | 16-18, I | Zb | Charge number for second particle |  |  |  |
|  | 19, I | Lpent | 1 to calculate penetrabilities 0 if penetrability = 1 |  | (required) |  |
|  | 20, I | Ishift | 1 to calculate shift 0 if S - B = 0.0 |  | (required) |  |
|  | 21-25, F | Sa | Spin and parity for P1 |  | (not needed if one of the special particle types is used, or if this is a fission pair) |  |
|  | 26-30, F | Sb | Spin and parity for P2 |  |  |  |
|  | 31-40, F | Ma | Mass of Particle 1 (amu) |  |  |  |
|  | 41-50, F | Mb | Mass of Particle 2 (amu) |  |  |  |
|  | 51-60, F | Ex | Excitation energy (eV) (Zero for ground state) |  | Asssumed to be defined in the laboratory system. Override with card-set 3 commands “CM NON COULOMB EXCITation” or “CM COULOMB EXCITATIOn”. |  |
|  | 61-70, F | Re | Effective radius for channels of this particle pair (fermi) |  | These are the “default” values, but they may be overwritten for some channels. |  |
|  | 71-80, F | Rt | True radius for for channels of this particle pair (fermi) |  |  |  |
| 4a: 2,3,etc. Repeat previous line once for each set of particle pairs |  |  |  |  |  |  |
| 4a: Last (end this card set with a blank line) |  |  |  |  |  |  |