| C:L | P, T | Variable Name | Meaning (units) | Notes |
| --- | --- | --- | --- | --- |
| 13:1 | 1-80, A | WHAT | “BACKGround functions” | See Section III.E.3.a. This card set could be put into the INPut file rather than PARameter file if no values are to be varied. |
| 13:2 | 1-5, A | WHAT | “CONST” Type 1 in Eq. (III E3 a.3) | Line is present only if needed and may be repeated as needed. |
|  | 7, I | IFA | Flag to vary or PUP A | 1 to vary, 3 to PUP, 0 otherwise |
|  | 11-20, F | A | Constant background (same units as data) |  |
|  | 21-30, F | dA | Uncertainty on background |  |
|  | 31-40, F | Emin | Minimum energy for which this background applies (eV) | Default energies are entire range of analysis. |
|  | 41-50, F | Emax | Maximum energy (eV) |  |
|  | 51-60, F | FPL | Flight-path length (m) | This value of the flight-path length is used only if DIST was not given in card set 5 of INPut file or elsewhere. |
| 13:3 | 1-5, A | WHAT | “EXPON” | This line is present only if needed, and may be repeated as needed. Type 2 in Eq. (III E3 a.3) |
|  | 7, I | IFA | Flag to vary or PUP A |  |
|  | 9, I | IFB | Flag to vary or PUP B |  |
|  | 11-20, F | A | Coefficient of exponential (units same as data) | See Section III.E.3.a for an explanation of this background function. |
|  | 21-30, F | dA | A priori uncertainty on A |  |
|  | 31-40, F | B | Coefficient of time in exp (ns) |  |
|  | 41-50, F | dB | A priori uncertainty on B |  |
|  | 51-60, F | Emin | Minimum energy for which this background applies (eV) | Default energies are entire range of analysis. |
|  | 61-70, F | Emax | Maximum energy (eV) |  |
|  | 71-80, F | FPL | Flight-path length (m) | This value of the flight-path length is used only if DIST was not given in card set 5 of the INPut file (or elsewhere). |
| 13:4 | 1-5, A | WHAT | “POWER” | This line is present only if needed and may be repeated as needed. Type 3 in Eq. (III E3 a.3) |
|  | 7, I | IFA | Flag to vary or PUP A |  |
|  | 9, I | IFB | Flag to vary or PUP B |  |
|  | 11-20, F | A | Coeff. of power of time (units are [data][ns-B] ) | See Section III.E.3.a for an explanation of this background function. |
|  | 21-30, F | dA | A priori uncertainty on A |  |
|  | 31-40, F | B | Power (dimensionless) |  |
|  | 41-50, F | dB | A priori uncertainty on B |  |
|  | 51-60, F | Emin | Minimum energy for which this background applies (eV) | Default energies are entire range of analysis. |
|  | 61-70, F | Emax | Maximum energy (eV) |  |
|  | 71-80, F | FPL | Flight-path length (m) | This value of L is used only if flight-path length was not given earlier. |
| 13:5 | 1-5, A | WHAT | “EXPLN” | This line is present only if needed and is always followed by line # 6. The pair of lines may be repeated as needed. Type 4 in Eq. (III E3 a.3). |
|  | 7, I | IFA | Flag to vary or PUP A |  |
|  | 9, I | IFB | Flag to vary or PUP B |  |
|  | 10, I | IFC | Flag to vary or PUP C |  |
|  | 11-20, F | A | Constant term in exponential (dimensionless) | See Section III.E.3.a for an explanation of this background function. |
|  | 21-30, F | dA | A priori uncertainty on A |  |
|  | 31-40, F | B | Coef. of t in exponential (ns−1) |  |
|  | 41-50, F | dB | A priori uncertainty on B |  |
|  | 51-60, F | C | Coefficient of [ln(t)] −1 in exponential (ln(nsec)) |  |
|  | 61-70, F | dC | A priori uncertainty on C |  |
| 13:6 | 11-20, F | Emin | Minimum energy for which this background applies (eV) | Line 6 must follow each and every occurrence of line 5. |
|  | 21-30, F | Emax | Maximum energy (eV) |  |
|  | 31-40, F | FPL | Flight-path length (m) | This value of L is used only if flight-path length was not given earlier. |
| 13:7 | 1-5, A | WHAT | “T-PNT” Type 5 in Eq. (III E3 a.5) | Point-wise linear function of time. If this line is present, it must occur at least twice. All occurrences must be contiguous in the file, and the times “T” must be increasing monotonically. |
|  | 7, I | IFA | Flag to vary or PUP A | 1 to vary, 3 to PUP, 0 otherwise |
|  | 11-20, F | A | Value of this background at time T (same units as data) | Section III.E.3.a provides an explanation of this background function. |
|  | 21-30, F | dA | Uncertainty on A |  |
|  | 31-40, F | T | Flight time for which this value of A applies (ns) |  |
|  | 41-50, F | FPL | Flight-path length (m) | This value of the flight-path length is used only if DIST was not given in card set 5 of INPut file or elsewhere. |
| 13:8 | 1-5, A, F | WHAT | “E-PNT” Type 6 in Eq. (III E3 a.6) | Point-wise linear function of energy. If this line is present, it must occur at least twice. All occurrences must be contiguous in the file, and the energies “E” must be increasing monotonically. |
|  | 7, I | IFA | Flag to vary or PUP A | 1 to vary, 3 to PUP, 0 otherwise |
|  | 11-20, F | A | Value of this background at energy E (same units as data) | Section III.E.3.a provides an explanation of this background function. |
|  | 21-30, F | dA | Uncertainty on A |  |
|  | 31-40, F | E | Energy for which this value of A applies (eV) |  |
| 13:9 | 1-5, A | WHAT | “TFILE” Type 7 in Eq. (III E3 a.7) | Point-wise linear function of time ti, with times and values Ai to be stored in a separate file. Line 10 must follow this line; the two lines may be repeated as needed. |
|  | 7, I | IFB | Flag to vary or PUP B | 1 to vary, 3 to PUP, 0 otherwise |
|  | 11-20, F | B | Multiplier for this background | Section III.E.3.a provides an explanation of this background function. |
|  | 21-30, F | dB | Uncertainty on B |  |
|  | 31-40, F | FPL | Flight-path length (m) | This value of the flight-path length is used only if DIST was not given in card set 5 of INPut file or elsewhere. |
| 13:10 | 1-70, A | FILE | Name of file containing the point-wise background. The first line in this file is a title, the other lines each contain values for two parameters (ti and Ai), in free format. | A file name on line 10 must follow every occurrence of line 9. |
| 13: 11 | 1-5, A | WHAT | “EFILE” Type 8 in Eq. (III E3 a.8) | Point-wise linear function of energy Ei, with times and values Ai to be stored in a separate file. Line 12 must follow this line; the two lines may be repeated as needed. |
|  | 7, I | IFB | Flag to vary or PUP B | 1 to vary, 3 to PUP, 0 otherwise |
|  | 11-20, F | B | Multiplier for this background | See Section III.E.3.a for details. |
|  | 21-30, F | dB | Uncertainty on B |  |
| 13:12 | 1-70, A | FILE | Name of file containing the point-wise background. The first line in this file is a title; the other lines each contain values for two parameters (Ei and Ai), in free format. | A file name on line 12 must follow every occurrence of line 11. |
| 13: 13 | 1-5, A | WHAT | “AETOB ” | This line is present only if needed, and may be repeated as needed. Type 9 in Eq. (III E3 a.9); |
|  | 7, I | IFA | Flag to vary or PUP A |  |
|  | 9, I | IFB | Flag to vary or PUP B |  |
|  | 11-20, F | A | Coefficient of E-B (units same as data) | See Section III.E.3.a for an explanation of this background function. |
|  | 21-30, F | dA | A priori uncertainty on A |  |
|  | 31-40, F | B | Exponent of E, where E is eV |  |
|  | 41-50, F | dB | A priori uncertainty on B |  |
|  | 51-60, F | Emin | Minimum energy for which this background applies (eV) | Default energies are entire range of analysis. |
|  | 61-70, F | Emax | Maximum energy (eV) |  |
| 13:Last (blank) |  |  |  |  |