III.C.3.b.  Input for the RPI/GELINA/nTOF resolution function

Input formats for all parameters of the RPI/GELINA/NTOF resolution function are given in Table VIB.2, card set 14.  Note that zero values are assumed to actually be zero and are not replaced by default values.  The first line of this card set contains one of the following phrases (which are treated as equivalent):

RPI Resolution function

GELINa resolution function

NTOF resolution function

As usual, only those characters in capital letters are required; those in lower case letters are optional.

In Table III C3 b.1, default values for the constants from Eqs. (III C3 a.2) through (III C3 a.5) are defined for transmission and capture measurements.  To invoke these defaults (without the option to flag them for varying), the user must specify the appropriate phrase

RPI Transmission resolution function or

RPI Capture resolution function

on the first line of card set 14.

Table III C3 b.2, provided courtesy of Frank Gunsing of Saclay [FG05], gives parameter values that may be suitable for use with data from the Geel Electron Linear Accelerator (GELINA) at IRMM and with data from the neutron time-of-flight facility (nTOF) at CERN.  Note that these values are preliminary; they are reasonable starting values which should be fine-tuned on measurements of well-known resonances (i.e., on resonances that have been measured many times at different installations).  To invoke these values automatically (without the option to flag them for varying), use one of the following phrases for the first line of card set 14:

GEEL DEFAUlt values for resolution parameters

GELINa DEFAUlt values

NTOF DEFAUlt values

With the RPI/GELINA/nTOF resolution function, it is necessary to specify whether the resolution function should be used exactly as given in the input or whether, instead, it should be shifted so that it is centered on the current energy.  To center the function, include one of the phrases

SHIFT RPI RESOLUTION function to center

SHIFT GEEL RESOLUTIOn function to center

SHIFT GELINA RESOLUtion function to center

SHIFT NTOF RESOLUTIOn function to center

in the command section of the INPut file.  To use the function exactly as given in the input, include one of the phrases

DO NOT SHIFT RPI RESolution function to center

DO NOT SHIFT GEEL Resolution function to center

DO NOT SHIFT GELINA resolution function to center

DO NOT SHIFT NTOF Resolution function to center

Many test cases provide examples of input for the RPI resolution function; see, for example, tr053, tr054, tr090, tr094, tr095, tr115, or tr136. For examples using the GELINA and nTOF resolution function, see test cases tr104, tr107, and (especially) tr108.

Table III C3 b.1.  Default values for parameters for RPI resolution function

|  | Parameter name | Value for transmission “RPI Transmission” | Value for capture a “RPI Capture” | Units |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | Ns |
| 2 | τ1 | 326. | 381. | Ns |
| 3 | τ2 | 0.0241 | 0.0058 | per eV |
| 4 | τ3 | 323. | 323. | Ns |
| 5 | τ4 | 0.029 | 0.094 | per eV |
| 6 | τ5 | 240. | 105. | Ns |
| 7 | τ6 | 0. | 0. | Ns |
| 8 | τ7 | 0. | 0. | [dimensionless] |
| 9 | Λ0 | 686.5 | 686.5 | Ns |
| 10 | Λ1 | -224.9 | -224.9 | Ns |
| 11 | Λ2 | 21.04 | 21.04 | ns |
| 12 | Λ3 | 0. | 0. | ns |
| 13 | Λ4 | 0. | 0. | [dimensionless] |
| 14 | a1 | -0.000985 | -0.001106 | per ns |
| 15 | a2 | 0.0241 | 0.0058 | per eV |
| 16 | a3 | -0.000626 | 0.04752 | per ns |
| 17 | a4 | 3.531 | 65.083 | per eV |
| 18 | a5 | 0.001029 | 0.001264 | per ns |
| 19 | a6 | 0. | 0. | per ns |
| 20 | a7 | 0. | 0. | [dimensionless] |
| 21 | t 0 | 940. | 940. | ns |
| 22 | A2 | -65.638 | -65.638 | [dimensionless] |
| 23 | A3 | 0.005 | 0.005 | per ns |
| 24 | A4 | 0.39383 | 0.39383 | [dimensionless] |
| 25 | A5 | 0.0008 | 0.0008 | per ns |
| next | Bi | 0.0 | 0.0 |  |
| last | c |  |  | ns |

a Values listed in this column are not necessarily those that are appropriate for the RPI capture resolution function, which has not yet been fully defined; these values are for illustrative purposes only.

Table III C3 b.2.  Parameters suitable for use with experimental data from GELINA at  IRMM and from nTOF at CERN.  Values are from preliminary work of F. Gunsing [FG05].  Parameters not listed in this table have value zero. Test case tr108 contains these values

| Parameter name | Value for GELINA 90-degree flight patha | Values for nTOF | Units |
| --- | --- | --- | --- |
| τ5 | -0.7722 | -3.7004 | ns |
| τ6 | 1363.85 | -684.39 | ns |
| τ7 | -0.5322 | -0.5189 | [dimensionless] |
| Λ0 | 1.4460 | 3.8457 | ns |
| Λ3 | 454.9720 | 502.9930 | ns |
| Λ4 | -0.5077 | -0.4155 | [dimensionless] |
| a1 | 0.0 | -0.0381 | per ns |
| a2 | 0.0 | 9.974×10-6 | per eV |
| a3 | 0.04152 | -0.01172 | per ns |
| a4 | -5.847×10-6 | 0.0001019 | per eV |
| a5 | -0.0415 | 0.05009 | per ns |
| a6 | 9.247×10-6 | 0.0 | per ns |
| a7 | 0.5961 | 0.0 | [dimensionless] |
| A2 | 1.0 | 1.0 | [dimensionless] |
| A4 | -1.0 | -1.0 | [dimensionless] |
| A3SQE (A3): a35 | 0.0003047 | -0.0001689 | per [ns ×(eV)1/2] |
| A3SQE (A3): a36 | 7.818×10-5 | 0.0004254 | per [ns ×(eV)1/2] |
| A3SQE (A3): a37 | -2.011 | -0.06043 | [dimensionless] |
| A3SQE (A5): a55 | 0.007331 | 0.0002766 | per [ns ×(eV)1/2] |

aThe GELINA resolution function refers to the beam only and does not include a contribution from a detection system.