Table VIB.1.  Header lines for card sets in the PARameter file

| Card set | Alphanumeric header line |  |  |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | Resonance parameters. No header line; card set ends in a blank line. |  |
| 2 |  |  |  |  | Fudge factor for default uncertainties; no header line, no blank line. |  |
| 3 | EXTERnal R-function parameters follow |  |  |  | Only one of card sets 3 and 3a is permitted. |  |
| 3a | R-EXTernal parameters follow |  |  |  |  |  |
| 4 | BROADening parameters may be varied |  |  |  |  |  |
| 5 | UNUSEd but correlated variables |  |  |  | Card set is created by SAMAMR run. |  |
| 6 | NORMAlization and background |  |  |  |  |  |
| 7 | RADIUs parameters follow (two versions: usual for fewer than 99 channels and spin groups, alternative version for greater than 99 channels or spin groups. |  |  |  | Card set 7a is the preferred method. |  |
| 7a | RADII are in KEY-WORD format or CHANNel radius parameters follow |  |  |  |  |  |
|  |  |  |  | Valid key words for channel radius: RADIus or RADIi EFfective radius TRue radius Valid key words for uncertainty: RElative ABsolute or UNcertainty Valid key words for particle-pair names: PARTicle-pair or PP Valid key words for orbital angular momentum: L or ORbital angular momentum with ODd, EVen, or ALl after equal sign Valid key word for flags: FLags with Yes, 1, No, 0, 2, or −1 Valid key word for spin groups: GRoup followed by a group number or All CHannels =, followed by channel numbers |  |  |
| 8 | DATA reduction parameters are next |  |  |  |  | Seldom used |
| 9 | ORRES |  |  |  |  |  |
|  |  | Valid subheadings for first five columns of subsequent lines: BURST WATER or TANTA (not both) LITHI or NE110 (not both) CHANN |  |  |  |  |
| 10 | ISOTOpic abundances and masses, or NUCLIde abundances and masses |  |  |  |  |  |
| 11 | MISCEllaneous parameters follow |  |  |  |  |  |
|  |  | Valid subheadings for first five columns of subsequent lines: DELTA ETA FINIT GAMMA TZERO SIABN SELFI EFFIC DELTE DRCAP |  |  |  |  |
| 12 | PARAMagnetic cross section parameters follow |  |  |  | In first five columns of next line, use one of “TM ”, “ER ”, “HO ” |  |
| 13 | BACKGround functions |  |  |  |  |  |
|  |  | Valid subheadings for first five columns of subsequent lines: CONST EXPON POWER EXPLN T-PNT E-PNT TFILE EFILE |  |  |  |  |
| 14 | RPI Resolution function, or GEEL resolution function, or GELINa resolution, or NTOF resolution function |  |  |  |  |  |
|  |  | Valid subheadings for first five columns of subsequent lines: BURST TAU LAMBD A1 EXPON A3 A5 XXPON BINS CHANN |  |  |  |  |
| 14a | RPI Transmission resolution function, or RPI Capture resolution function, or GEEL DEFAUlts, or GELINa DEFAUlts, or NTOF DEFAUlts |  |  |  |  |  |
|  |  | Valid subheadings for first five columns of subsequent lines: BURST BINS CHANN |  |  |  |  |
| 15 | DETECtor efficiencies |  |  |  |  |  |
| 16 | USER-Defined resolution function |  |  |  |  |  |
|  |  | Valid subheadings for first five columns of subsequent lines: BURST CHANN FILE= |  |  |  |  |
| Last | COVARiance matrix is in binary form in another file |  |  |  |  |  |
| Last B | EXPLIcit uncertainties and correlations follow |  |  |  |  |  |
| Last C | RELATive uncertainties follow |  |  |  |  |  |
| Last D | PRIOR uncertainties follow in key-word format See Table VI B.2 for details. |  |  |  |  |  |
|  |  |  | Valid key words are: RELAtive, ABSOlute, or UNCErtainty EMIN or EMAX GRoup with number or All Channel with number or All Energy or Gamma PP or PARTicle pair L or Orbital angular momentum |  |  |  |