IV.E.4.   Covariances for Calculated Cross Sections

Let Ti represent the theoretical cross section (or transmission), at energy point Ei, as calculated from the “final” set of resonance parameters and parameter covariance matrix.  The covariance matrix element connecting theory point Ti with point Tj is found from

where Mkn is the covariance matrix element connecting parameter uk with parameter un .  The theoretical uncertainty on theory Ti is the square root of the diagonal element; that is,

When the phrase

INCLUDE THEORETICAL uncertainties in plot file

is given in the command section of the INPut file, values of ΔT i will be reported in the plot file.  See Section VII.C for specific details about where to find which uncertainties.

To create an ASCII listing of cross section (or transmission), uncertainty, and correlation matrix, include the phrase

CROSS SECTION COVARIance matrix is wanted

in the command section of the INPut file.  The desired listing will be given in the SAMMY.LPT file. This is a no-Bayes run (i.e., no fitting can be accomplished in the same SAMMY run).

Using this command will also give the cross-section covariance matrix formatted into a “publishable” file named SAMCOV.PUB.  This file is formatted with values separated by tabs so that it can be easily ported to a spreadsheet or other program, for example, to be plotted in three dimensions.  This SAMCOV.PUB file contains the following information:

Row 1:  three tabs, followed by the Ti separated by tabs

Row 2:  three tabs, followed by the ΔTi separated by tabs

Row 3:  three tabs, followed by the Ei separated by tabs

Row 4:  T1, Δ T1, E1, followed by the first row of the covariance matrix separated by tabs

Row 5:  T2, Δ T2, E2, followed by the second row of the covariance matrix separated by tabs

. . .

Row N+3: TN, Δ TN, EN, followed by the Nth row of the covariance matrix separated by tabs

In this format, it is easily possible to manipulate the results in a spreadsheet program. For example, the correlation matrix is found by dividing the covariance matrix element by the uncertainty in the same column in Row 2 and by the uncertainty in the same row in Column 2.  See test case tr010, run e, for an example of this.  (Caution:  The user should exercise caution in porting this file to a spreadsheet, as the number of data points can easily become too large to fit into a normal spreadsheet.)

The SAMMY user should note that the covariance matrix constructed in this fashion is not a complete representation of the evaluated cross section covariance matrix.  Detail arguments concerning this assertion are presented in Section IV.E.6.