| Category | D | Statements | Notes | # |
| --- | --- | --- | --- | --- |
| Special options |  | RELEVANT PARAMETERS are chosen via uncertainty ratio | Solve Bayes’ equations one time. Then, based on the ratio of u-parameter uncertainties, choose which of the R-matrix parameters are relevant. Create file SAMMY.REL, which contains the original parameter values with only the relevant parameters flagged to be varied. See test case tr166 for examples. | 327 |
|  |  | CROSS SECTION COVARIance matrix is wanted | Calculate the covariance matrix for the theoretical cross sections. | 328 |
|  |  | INITIAL UNCERTAINTY multiplier = | All prior parameter uncertainties (square roots of diagonal elements of parameter covariance matrix) are to be multiplied by the specified number. See Section IV.E.6 for further discussion, and test case tr019 for examples. | 329 |
|  |  | FINAL UNCERTAINTY MUltiplier = | All output parameter uncertainties (square roots of diagonal elements of parameter covariance matrix) are to be multiplied by the specified number. See Section IV.E.6 for further discussion, and test case tr019 for examples. | 330 |
|  |  | E-DEPENDENT INITIAL Uncertainty multiplier, or E-DEPENDENT UNCERTAInty multiplier | All prior parameter uncertainties are to be multiplied by an energy- and spin-group-dependent multiplier. Details are give in Section IV.E.6, and test case tr019 contains examples. | 6 7 |
|  |  | SUMMED STRENGTH FUNCtion is wanted | Evaluate and print the summed strength function and corresponding covariance matrix. See Section V.F for details. | 331 |
| Special options (cont.) |  | GENERATE PARTIAL DERivatives only | SAMMY will assume all resonance parameters, and only resonance para-meters, are to be varied; that Bayes' equations are not to be solved; and that the partial derivatives of the theoretical values with respect to those parameters are to be written into file SAMMY.PDS . | 332 |
|  |  | GENERATE SPIN GROUP cross sections | Generate the ODF (Plot) file containing the energy (from auxiliary grid) in section 1, the unbroadened cross section in section 2, the spin-group-1 contribution to the unbroadened cross section in section 3, spin-group-2 contribution in section 4, etc. | 333 |
|  |  | REFORMULATE DATA FOR implicit data covariances | See Sections IV.D.3 and VI.C.3. | 334 |
|  |  | COMPARE EXPERIMENT To theory | See Section X.E; results are reported in binary file SAM53.DAT. | 335 |
|  |  | GENERATE Y AND W MATrices, or YW | See Section IV for a description of the M+W form of Bayes’ Equations. See test case tr089 for examples of the use of this option. | 336 337 |
|  |  | READ Y AND W MATRICEs, or WY |  | 338 339 |
|  |  | STOP abc n (Note that there are two spaces before “n” in this command; n is replaced with an integer value.) | Cease execution prior to the nth occurrence of segment abc. Used only for debug (Section XI.B) and for preparation for Monte Carlo calculations (Section X.M). | 340 |
|  |  | WRITE CALCULATED CROss sections into ascii file | Write energies and theoretical values into ascii file SAMTHE.DAT in 2F20.10 format. | 341 |
| Special options (cont.) |  | PRINT MULTIPLE SCATTering corrections | Create a table of E, Y0, Y1, Y2, and Y (energy plus components of the corrected yield) in file SAM012.DAT. | 275 |
|  |  | UNIFORM ENERGY GRID | Instead of using the energies from the experimental data set to define the energy grid, SAMMY will create a grid of 1001 points uniformly space in energy. This is for testing purposes only, so this grid must be used only for “no Bayes” runs. See Section V.E for details. | 342 |
|  |  | UNIFORM VELOCITY GRId | SAMMY will create a grid of 1001 points uniformly spaced in velocity (square root of energy). To be used for testing purposes only. See Section V.E. | 343 |
|  |  | UNIFORM TIME GRID | SAMMY will create a grid of 1001 points uniformly spaced in time (1 over the square root of energy). To be used for testing purposes only. See Section V.E for details. | 344 |
|  |  | CREATE PUBLISHABLE List of parameters, or PUBLISH | The file SAMMY.PUB will be created, which contains resonance parameters and uncertainties in columns, with each value separated by tabs. This file can be ported to spreadsheet programs to be formatted for publication purposes. See Section VII.F for details. | 345 346 |
|  |  | DO NOT TEST FOR EIGEnvalues | Do not test whether the parameter covariance matrix is positive-definite (i.e., whether all eigenvalues are greater than zero). | 347 |