| Category | D | Statements | Notes | # |
| --- | --- | --- | --- | --- |
| R-matrix approx-imation | D | REICH-MOORE FORMALISm is wanted, or MORE ACCURATE REICH- moore, or XCT | See Section II. Note that “more accurate” refers to the computational method, not to the formalism. | 171 172 173 |
|  |  | ORIGINAL REICH-MOORE formalism, or CRO | See Section II. This method of calculating the RM formalism is sometimes inaccurate, particularly at low energies for capture. Probably this option will be eliminated in later releases of the code. | 174 175 |
|  |  | MULTILEVEL BREIT- WIGner is wanted, or MLBW FORMALISM IS WAnted, or MLBW | See Section II.B.3. Note that SAMMY’s MLBW treatment is identical to that specified in ENDF-102; that is, only elastic cross sections are MLBW, while fission and capture are calculated via SLBW equations. | 176 177 178 |
|  |  | SINGLE LEVEL BREIT- Wigner is wanted, or SLBW FORMALISM IS WAnted, or SLBW | See Section II.B.3. | 179 180 181 |
|  |  | REDUCED WIDTH AMPLITudes are used for input | Card set 1a of the PARameter file is to be used for input of resonance parameters. | 184 |
|  |  | (No command) | When “type of cross section” (see card set 8 of Table VI A.1) is “1/v”, “constant”, “linear”, “quadratic”, or “Dirac delta function”, then that designated type is calculated; this overrides any command given in the alphanumeric listing for Reich-Moore, etc. This option is used for debug only; see Section V.E for more details. |  |