| C:L | P,T | Variable Name | Meaning (units) | Notes |
| --- | --- | --- | --- | --- |
| 11:5 | 1-5, A | WHAT | “GAMMA” | Use the same Γγ for all resonances in this spin group. If this line exists for any spin group, then a line must be given for every spin group. |
|  | 6-7, I | IG | Spin group number |  |
|  | 8-9, I | IFG | Flag for GAMGAM |  |
|  | 11-20, F | GAMGAM | Radiation width Γγ for all resonances in spin group IG |  |
|  | 21-30, F | DGAM | Uncertainty on GAMGAM |  |
| 11:6 | 1-5, A | WHAT | “TZERO” | See Section III.E.8. |
|  | 7, I | IFTZER | Flag for t 0 |  |
|  | 9, I | IFLZER | Flag for L 0 |  |
|  | 11-20, F | TZERO | t 0 (μs) |  |
|  | 21-30, F | DTZERO | Uncertainty on t 0 (μs) |  |
|  | 31-40, F | LZERO | L 0 (dimensionless) |  |
|  | 41-50, F | DLZERO | Uncertainty on L 0 (dimensionless) |  |
|  | 51-60, F | FPL | Flight-path length (m) | This value for the flight-path length is used only if DIST is zero ( INPut File, card set 5). |
| 11:7 | 1-5, A | WHAT | “SIABN” | Abundances for self-indication transmission sample. See Section III.E.6. |
|  | 7, I | IF1 | Flag for SIABN(1) |  |
|  | 9, I | IF2 | Flag for SIABN(2) |  |
|  | 10, I | IF3 | Flag for SIABN(3) |  |
|  | 11-20, F | SIABN(1) | Abundance for nuclide # 1 | Note that nuclides are defined in card set 10, which must occur before card set 11 in the PARameter file. Repeat this line until all nuclides have been included. |
|  | 21-30, F | DS(1) | Uncertainty on SIABN(1) |  |
|  | 31-40, F | SIABN(2) | Abundance for nuclide # 2 |  |
|  | 41-50, F | DS(2) | Uncertainty on SIABN(2) |  |
|  | 51-60, F | SIABN(3) | Abundance for nuclide # 3 |  |
|  | 61-70, F | DS(3) | Uncertainty on SIABN(3) |  |
| 11:8 | 1-5, A | WHAT | “SELFI” | Temperature and thickness for transmission sample for self-indication experiment. See Section III.E.6. |
|  | 7, I | IFTEMP | Flag for temperature |  |
|  | 9, I | IFTHCK | Flag for thickness |  |
|  | 11-20, F | SITEM | Effective temperature (K) for self-indication transmission sample |  |
|  | 21-30, F | dSITEM | Uncertainty on SITEM |  |
|  | 31-40, F | SITHC | Thickness (atoms/barn) |  |
|  | 41-50, F | dSITHC | Uncertainty on SITHC |  |
| 11:9 | 1-5, A | WHAT | “EFFIC” | Efficiencies for capture and for fission for eta calculation. |
|  | 7, I | IFCAPE | Flag for capture efficiency |  |
|  | 9, I | IFFISE | Flag for fission efficiency |  |
|  | 11-20, F | EFCAP | Efficiency for detecting capture events |  |
|  | 21-30, F | EFFIS | Efficiency for detecting fission events |  |
|  | 31-40, F | dEFCAP | Uncertainty on EFCAP |  |
|  | 41-50, F | dEFFIS | Uncertainty on EFFIS |  |
| 11:10 | 1-5, A | WHAT | “DELTE” | Delta-E is energy dependent, as described in Section III.C.1.b, Eq. (III C1 b.7). |
|  | 7, I | IFLAG1 | Flag for DELE1 |  |
|  | 9, I | IFLAG0 | Flag for DELE0 |  |
|  | 10, I | IFLAGL | Flag for DELEL |  |
|  | 11-20, F | DELE1 | Coefficient of E in Eq. (III A2b.7) (m / eV) |  |
|  | 21-30, F | DD1 | Uncertainty on DELE1 |  |
|  | 31-40, F | DELE0 | Constant term in Eq. (IIIA2b.7) (m) |  |
|  | 41-50, F | DD0 | Uncertainty on DELE0 |  |
|  | 51-60, F | DELEL | Coefficient of log term in Eq. (IIIA2b.7) (m / ln(eV)) |  |
|  | 61-70, F | DDL | Uncertainty on DELEL |  |
| 11:11 | 1-5, A | WHAT | “DRCAP” | Numerical direct capture component for nuclide number NUC is read from DRC file; COEF (NUC) multiplies that value. Note that this line may be included as many times as needed, to a maximum of once per nuclide. See Section II.B.4 for details. |
|  | 7, I | IFLAG1 | Flag to vary COEF(Nuc) |  |
|  | 9, I | NUC | Nuclide Number |  |
|  | 11-20, F | COEF | Coefficient of the value given in the DRC file |  |
|  | 21-30, F | dCOEF | Uncertainty on COEF |  |
| 11:12 | 1-5, A | WHAT | “NSENS” | This line must be present when neutron sensitivity calculations are to be included with self-shielding and multiple-scattering corrections. See Section III.D, Eq. (III D.17), for details. |
|  | 7, I | IFNEUT | Flag for neutron sensitivity |  |
|  | 11-20, F | SENSIN | A = multiplier for neutron-sensitivity calculation; value is equal to the binding energy of the target nucleus. (SAMMY will automatically set this to a positive value, even if a negative value is given.) |  |
|  | 21-30, F | DSENSN | Uncertainty on A |  |
| 11:13 | 1-5,A | WHAT | “NONUN” or “NON U” | Non-uniform sample; thickness is a function of radius (see Section III.E.1.a for transmission). At least two lines must be given, all together in one location in the card set; the first line must have zero radius (the center of the sample) and the last must be at the outer edge of the sample. No fitting is yet permitted with these parameters. See test case tr178 for examples. |
|  | 21-30, F |  | Radius at which thickness is to be given |  |
|  | 31-40, F |  | Positive value for the sample thickness at this radius |  |
|  | 41-50, F |  | Uncertainty on thickness |  |
| 11:Last (blank) |  |  |  |  |