Table X C.1.  Commands to analyze data from three samples of different

thicknesses at two different temperatures

| Interactive input | Description of what is being done |  |
| --- | --- | --- |
| sammy <<EOF t007a.inp t007a.par t007a.dat EOF |  | analyze first data set using TEMP1 & THCK1 |
| mv -f SAMMY.LPT results/raa.lpt mv –f SAMMY.ODF results/raa.odf mv –f SAMMY.PAR results/raa.par mv –f SAMMY.COV results/raa.cov |  | rename the output files |
| samamr <<EOF a results/raa.par results/raa.cov results/rcc.par results/rcc.cov TEMP TEMP1 181.0 10.0 EOF |  | mothball TEMP1; insert TEMP2 |
| samamr <<EOF a results/rcc.par results/rcc.cov results/rdd.par results/rdd.cov THICK THCK1 0.01754 0.0001754 EOF |  | mothball THCK1; insert THCK2 |
| sammy <<EOF t007d.inp results/rdd.par t007e.dat results/rdd.cov EOF |  | analyze second data set with TEMP2 & THCK2; with UNUSED #1=TEMP1, #2=THCK1 |
| mv –f SAMMY.LPT results/ree.lpt mv –f SAMMY.PAR results/ree.par mv –f SAMMY.ODF results/ree.odf mv –f SAMMY.COV results/ree.cov |  |  |
| samamr <<EOF m results/ree.par results/ree.cov results/rff.par results/rff.cov TEMP TEMP2 1 EOF |  | interchange so USED=TEMP1, UNUSED#1=TEMP2 |
| samamr <<EOF a results/rff.par results/rff.cov results/rgg.par results/rgg.cov THICK THCK2 0.01319 0.0001319 EOF |  | mothball THCK2, insert THCK3 |
| sammy <<EOF # tr7hhh t007h.inp results/rgg.par t007h.dat results/rgg.cov EOF |  | analyze third data set with TEMP1 & THCK3; with UNUSED #1=TEMP2, #2=THCK1, #3=THCK2 |
| mv SAMMY.LPT results/rhh.lpt mv SAMMY.PAR results/rhh.par mv SAMMY.ODF results/rhh.odf mv SAMMY.COV results/rhh.cov |  | rename files |
| samamr <<EOF m results/rhh.par results/rhh.cov results/rii.par results/rii.cov THICK THCK3 2 EOF |  | interchange THICK and UNUSED#2 (so will use TEMP1 & THCK1) |
| sammy <<EOF t007j.inp results/rii.par t007a.dat results/rii.cov EOF |  | generate cross section for first data set with TEMP1 & THCK1; with UNUSED #1=TEMP2, #2=THCK3, #3=THCK2 |
| mv SAMMY.LPT results/rjj.lpt mv SAMMY.ODF results/rjj.odf |  | rename files |
| samamr <<EOF m results/rii.par results/rii.cov results/rkk.par results/rkk.cov TEMP TEMP1 1 EOF |  | interchange TEMP=TEMP1 with UNUSED#2=TEMP2 |
| samamr <<EOF m results/rkk.par results/rkk.cov results/rll.par results/rll.cov THICK THCK1 3 EOF |  | interchange THICK=THCK1 with UNUSED#3=THCK2 |
| sammy <<EOF t007m.inp results/rll.par t007e.dat results/rll.cov EOF |  | generate cross section for second data set with TEMP2 & THCK2; with UNUSED #1=TEMP1, #2=THCK3, #3=THCK1 |
| mv SAMMY.LPT results/rmm.lpt mv SAMMY.ODF results/rmm.odf |  | rename files |