III.B.1.a.   Derivatives with respect to effective temperature T

In SAMMY, the Doppler temperature T may be treated as a u-parameter (see Section IV)  and varied via Bayes= equations.  Thus, it is necessary to determine derivatives of the broadened cross section (or transmission) with respect to T.

The Doppler-broadening integral of Eq. (III B1.7) depends on effective temperature T only through the Doppler width ΔD of Eq. (III B1.2).  Using the chain rule we find

where the first of these partial derivatives is found from Eq. (III B1.2) to be

The partial derivative of the cross section with respect to the Doppler width is evaluated numerically in SAMMY, via

The value of d to use in this expression is somewhat arbitrary, subject to the restrictions that (1) d be small enough that  is approximately a linear function of Δ in the region  and (2) d be big enough that sufficient significant digits are retained by the computer in generating the numerical difference in .  Through trial and error, it was determined that setting d equal to ΔD q, with q = 0.02, gives reasonable results for those cases tested.  Since the value of this parameter q is not subject to direct user control, anyone experiencing difficulty when varying the effective temperature should contact the author.