| Command statement | Category | Parameter | Default | deBug | Archaic |
| --- | --- | --- | --- | --- | --- |
| ADD CONSTANT TERM TO data covariance | DCM |  |  |  |  |
| ADD CROSS SECTIONS From endf/b file 3 | Average |  |  |  |  |
| ADD DIRECT CAPTURE Component… | CS calc |  |  |  |  |
| ADD ELIMINATED CAPTUre channel to final state | CS calc |  |  |  |  |
| ADD TEN PERCENT DATA uncertainty | DCM |  |  |  |  |
| ANGLE-AVERAGE FOR DIfferential cross section | Angle |  |  |  |  |
| ANNOTATED | URR |  | D |  |  |
| ANNOTATED PARAMETER file for urr | URR |  | D |  |  |
| APPROXIMATE SCATTEREd neutron attenuation… | Angle |  |  |  |  |
| ARTIFICIAL ENERGY GRid is needed | Grid |  |  |  |  |
| AUTOMATIC NDF FILE Creation | ENDF out |  |  |  |  |
| AVERAGE OVER ENERGY ranges | Average |  |  |  |  |
| BONDARENKO AVERAGE Over energy ranges | Average |  |  |  |  |
| BROADENING IS NOT WAnted | Broad |  |  |  |  |
| BROADENING IS WANTED | Broad |  | D |  |  |
| CALCULATE DRESNER INtegrals more accurately | URR |  |  |  |  |
| CALCULATE MAXWELLIAN averages after reconst… | Average |  |  |  |  |
| CALCULATE WIDTH FLUCtuation factors more… | URR |  |  |  |  |
| CENTER THE CONSTANT energy resolution function | Resol |  |  |  |  |
| CHI SQUARED IS NOT Wanted | LPT |  |  |  |  |
| CHI SQUARED IS WANTEd | LPT |  | D |  |  |
| CLM | Doppler |  |  |  |  |
| CM COULOMB EXCITATIOn energies | CS calc |  |  |  |  |
| CM NON COULOMB EXCITation energies | CS calc |  |  |  |  |
| COMPACT CORRELATIONS are to be read and used | PCM in |  |  |  |  |
| COMPACT COVARIANCES are to be read and used | PCM in |  |  |  |  |
| COMPARE EXPERIMENT To theory | Special |  |  |  |  |
| CREATE PUBLISHABLE List of parameters | Special |  |  |  |  |
| CREATE PUP FILE FROM varied parameters used… | DCM |  |  |  |  |
| CRO | RM |  |  |  | A |
| CROSS SECTION | MSC |  |  |  |  |
| CROSS SECTION COVARIance matrix is wanted | Special |  |  |  |  |
| CSISRS | Data |  |  |  |  |
| DATA ARE ENDF/B FILE | Data |  |  |  |  |
| DATA ARE IN ODF FILE | Data |  |  |  |  |
| DATA ARE IN ORIGINAL multi-style format | Data |  | D |  |  |
| DATA ARE IN STANDARD odf format | Data |  |  |  |  |
| DATA COVARIANCE FILE is named YYYYYY.YYY | DCM | P |  |  |  |
| DATA COVARIANCE IS Diagonal | DCM |  | D |  |  |
| DATA FORMAT IS ONE Point per line | Data |  |  |  |  |
| DATA HAS OFF-DIAGONAl contribution to cov... | DCM |  |  |  |  |
| DEBUG | LPT |  |  | B |  |
| DIFFERENTIAL DATA ARe In ascii file | Data |  |  |  |  |
| DIFFERENTIAL DATA ARe in ascii file | Angle |  |  |  |  |
| DIVIDE DATA INTO REGions with a fixed number… | Data |  |  |  |  |
| DO NOT ADD CONSTANT term to data covariance | DCM |  | D |  |  |
| DO NOT ALLOW VALUES for data-related param… | Special |  |  |  |  |
| DO NOT DIVIDE DATA Into regions | Data |  | D |  |  |
| DO NOT GENERATE PLOT file automatically | Plot |  | D |  |  |
| DO NOT INCLUDE SELF-shielding or multiple-scat… | MSC |  | D |  |  |
| DO NOT INCLUDE THEORetical uncertainties in… | Plot |  |  |  |  |
| DO NOT PRINT ANY INPut parameters | LPT |  | D |  |  |
| DO NOT PRINT BAYES Chi squared | LPT |  |  |  |  |
| DO NOT PRINT BAYES Weighted residuals | LPT |  | D |  |  |
| DO NOT PRINT DEBUG Info | LPT |  | D |  |  |
| DO NOT PRINT INPUT Data | LPT |  |  |  |  |
| DO NOT PRINT LS CHI squared | LPT |  |  |  |  |
| DO NOT PRINT LS WEIGhted residuals | LPT |  | D |  |  |
| DO NOT PRINT PARTIAL derivatives | LPT |  | D |  |  |
| DO NOT PRINT PHASE Shifts | LPT |  | D |  |  |
| DO NOT PRINT REDUCED widths | LPT |  | D |  |  |
| DO NOT PRINT SMALL Correlation coefficients | LPT |  | D |  |  |
| DO NOT PRINT THEORETical values | LPT |  |  |  |  |
| DO NOT PRINT WEIGHTEd residuals | LPT |  | D |  |  |
| DO NOT SHIFT ENERGY for exponential tail… | Resol |  |  |  |  |
| DO NOT SHIFT GEEL REsolution function… | Resol |  |  |  |  |
| DO NOT SHIFT GELINA resolution function… | Resol |  |  |  |  |
| DO NOT SHIFT NTOF REsolution function… | Resol |  |  |  |  |
| DO NOT SHIFT RPI RESolution function… | Resol |  |  |  |  |
| DO NOT SOLVE BAYES Equations | Bayes |  |  |  |  |
| DO NOT SUPPRESS ANY intermediate printout | LPT |  | D |  |  |
| DO NOT SUPPRESS INTErmediate printout | LPT |  |  |  |  |
| DO NOT TEST FOR EIGEnvalues | Special |  |  |  |  |
| DO NOT USE ENERGY LImits as given in the… | URR |  |  |  |  |
| DO NOT USE SHORT FORmat for output | LPT |  | D |  |  |
| DO NOT USE S-WAVE CUtoff | CS calc |  | D |  |  |
| DOUBLE | MSC |  |  |  |  |
| E-DEPENDENT INITIAL uncertainty multiplier | Special |  |  |  |  |
| E-DEPENDENT UNCERTAInty multiplier | Special |  |  |  |  |
| ENDF | ENDF out |  |  |  |  |
| ENDF | URR |  |  |  |  |
| ENDF COVARIANCE MATRix is to be read and used | PCM in |  |  |  |  |
| ENDF/B-VI FILE 2 IS wanted | ENDF out |  |  |  |  |
| ENDF/B-VI FILE 2 IS wanted | URR |  |  |  |  |