| Card set | Meaning (units) |  | Notes |
| --- | --- | --- | --- |
| The remaining card sets in the INPut file are exact duplicates of corresponding card sets in the PARameter file. Any given card set can be in at most one location (i.e., it may be either in the INPut file, in the PARameter file, or in neither, but not in both). A flag to vary a parameter (flag = 1) is ignored when the card set is in the INPut file. A flag equal to 3 will, however, cause the parameter to be PUP’d (see Section IV.D.2 for a discussion of propagated uncertainty parameters). If no multiple-scattering size parameters (card set 11) are given in the INPut file, two blank lines must precede the beginning of these card sets. The ordering of these card sets does not matter; however, they must follow everything else in the INPut file. |  |  |  |
| P4 | BROADening parameters |  |  |
| P6 | NORMAlization and background |  | Section III.E.3.a |
| P8 | DATA reduction parameters |  | See Section III.E.3.b |
| P9 | ORRES (Oak Ridge Resolution Function ) |  | Section III.C.2 |
| P10 | ISOTOpic or NUCLIde abundances |  |  |
| P11 | MISCEllaneous parameters |  |  |
| P12 | PARAMagnetic cross section parameters |  | Section III.E.4 |
| P13 | BACKGround functions |  | Section III.E.3.a |
| P14 | RPI R (RPI Resolution Function ), or GEEL , or GELINa, or NTOF |  | Section III.C.3 |
| P14a | RPI T or RPI C (note the blank space after “RPI” in both cases) GEEL DEFAUlts, or GELINa DEFAUlts, or NTOF DEFAUlts |  | RPI Resolution Function with either the relevant default values Section III.C.3 |
| P15 | DETECtor efficiencies. |  | Section III.E.5 |
| P16 | USER- (USER-Defined Resolution Function) |  | Section III.C.5 |