| Category | D | Statements | Notes | # |
| --- | --- | --- | --- | --- |
| Options for printing into SAMMY. LPT | D | DO NOT PRINT ANY INPut parameters |  | 91 92 |
|  |  | PRINT ALL INPUT PARAmeters | All resonances parameters are printed in SAMMY.LPT. | 93 |
|  |  | PRINT VARIED INPUT Parameters | Resonances for which no parameters are flagged will not be printed. Use this option for direct comparison with output parameters, which are always printed in this mode. | 94 |
|  | D | DO NOT PRINT INPUT Data | “Input data” refers to experimental values of the measured cross section (or transmission, etc.), as read from the DATa file (see Section VI.C). “Theoretical values” refers to the calculated cross sections (as broadened and otherwise corrected). Often these are available in the plot file (see Table VII C.1), so they need not be printed in the LPT file. | 95 |
|  |  | PRINT INPUT DATA,or PRINT EXPERIMENTAL Values |  | 96 97 |
|  | D | DO NOT PRINT THEORETical values |  | 98 |
|  |  | PRINT THEORETICAL VAlues, or PRINT THEORETICAL CRoss sections |  | 99 100 |
|  | D | DO NOT PRINT PARTIAL derivatives |  | 101 |
|  |  | PRINT PARTIAL DERIVAtives | Use this option only for debug purposes. | 102 |
|  |  | SUPPRESS INTERMEDIATe printout | Updated parameter values and covariance matrix elements are printed only upon completion of entire run. | 103 |
|  |  | DO NOT SUPPRESS INTErmediate printout | Updated parameter values and covariance matrix elements are printed after completion of each energy region. | 104 |
|  | D | DO NOT SUPPRESS ANY intermediate printout | Updated parameter values are printed after each iteration of Bayes' equations (i.e., typically twice for each energy region). Updated covariance matrix elements are evaluated and printed only upon completion of an energy region. | 105 |
| Options for printing into SAMMY. LPT (cont.) | D | DO NOT USE SHORT FORmat for output |  | 106 |
|  |  | USE SHORT FORMAT FOR output | Resonance parameters are printed in the LPT file (Section VII.A), in F12.4 format rather than E format. Although this may produce more legible output, fewer significant digits may be printed. | 107 |
|  | D | DO NOT PRINT REDUCED widths |  | 108 |
|  |  | PRINT REDUCED WIDTHS | Reduced width amplitudes will be printed in the LPT file (Section VII.A), along with the square root of the resonance energies. That is, what are printed are the “u-parameters” as described in Section IV.C. | 109 |
|  |  | DO NOT PRINT SMALL Correlation coefficients | Any line of the correlation matrix whose off-diagonal elements are small will not be printed. “Small” is defined as less than ICORR/100 in absolute value, where ICORR is given in card set 2 of Table VI A.1. | 110 |
|  | D | DO NOT PRINT DEBUG Info |  | 112 |
|  |  | PRINT DEBUG INFORMATion, or DEBUG | Use for debug purposes only on short runs. | 113 114 |
|  |  | PRINT CAPTURE AREA In lpt file | The capture area, defined as , is printed in the SAMMY.LPT file. | 115 |
|  |  | CHI SQUARED IS NOT Wanted, or DO NOT PRINT LS CHI squared | See Section IV.A.2 for definitions of bothand . Independent of which commands are given, both values are printed whenever they can be calculated without excess effort. | 116 117 |
|  | D | CHI SQUARED IS WANTEd, or PRINT LS CHI SQUARED |  | 118 119 |
|  | D | PRINT BAYES CHI SQUAred |  | 121 |
|  |  | DO NOT PRINT BAYES Chi squared |  | 120 |
| Options for printing into SAMMY. LPT (cont.) | D | DO NOT PRINT WEIGHTEd Residuals, or DO NOT PRINT LS WEIGhted residuals | See Section IV.A.2 for definitions of weighted residuals. | 122 123 |
|  |  | PRINT WEIGHTED RESIDuals, or PRINT LS WEIGHTED REsiduals |  | 124 125 |
|  |  | PRINT BAYES WEIGHTED residuals |  | 126 |
|  | D | DO NOT PRINT BAYES Weighted residuals |  | 127 |
|  | D | DO NOT PRINT PHASE Shifts |  | 130 |
|  |  | PRINT PHASE SHIFTS For input parameters | Hard-sphere phase shifts are defined in Section II.A. | 131 |