| C:L | P,T | Variable name | Meaning (units) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 11: 1 | This card set is present only if self-shielding and multiple-scattering corrections are mentioned in card set 3. See Section III.D for details. |  |  |  |  |
|  | 1-10, F | THSAMP | Sample thickness (cm) This may be calculated as the thickness in atoms/barn, multiplied by the atomic mass in amu, multiplied by the inverse of Avagadro’s number (i.e., by 1.66053886 in the appropriate units), divided by the density for the material in g/cm3. See, for example, the web site http://www.chemicool.com/ for values for density. |  |  |
|  | 11-20, F | XSAMP | Sample height (cm) or sample radius (cm) |  |  |
|  | 21-30, F | YSAMP |  |  | Currently unused |
|  | 31-40, F | XBEAM | Beam height (cm) or beam radius (cm), or zero if beam is larger than sample |  | If XSAMP is given as zero here but XBEAM is positive, the code will set XSAMP = XBEAM, and vice versa. |
|  | 41-50, F | YBEAM |  |  | Currently unused |
|  | 51-55, I | NTHETA | Total number of points for theta integration |  | Default = 33 |
|  | 56-60, I | NGAUSZ | Number of points for pieces of z integration |  | 4, 8, or 16 Default = 16 |
|  | 61-65, I | NGAUS | Number of points for pieces of integration over cross section of beam |  | 4, 8, or 16 Default = 16 |
|  | 66-70, I | MTHETA | Number of points for theta integration near the end points (2*MTHETA < NTHETA) |  | Default = 5 |
|  | 71-75, I | NXTPT | Number of points for integration and/or interpolation over σ; see Eq. (III D.8) and following |  | Default = 32 |
|  | 76-80, I | MXTPT | Number of points for integration over σ'; see Eq. (III D.8) ff |  | Default = 32 |