| Category | D | Statements | Notes | # |
| --- | --- | --- | --- | --- |
| Broadening – resolution |  | TWO RESOLUTION FUNCTions together | It is possible to use more than one type of resolution function in the same run. See Section III.C. | 231 |
|  |  | USE TRUE TOTAL CROSS section for resolution broadening | Usually total cross sections will be converted to transmissions after Doppler broadening, before resolution broadening. This command forces the use of the total cross section rather than the transmission for resolution broadening and is useful in comparing the true total cross section with the effective total cross section. See Section III.E.1 for details, and test case tr081 for an example. | 232 |
|  |  | EXPONENTIAL FOLDING width is energy- dependent | See Section III.C.1.b, Eq. (III C1 b.6). | 233 |
|  | D | SHIFT ENERGY FOR EXPonential tail on resolution broadening | See Section III.C.1.c; the energy shift ΔEs is chosen to give a maximum of the integrand at the resonance energy. | 234 |
|  |  | DO NOT SHIFT ENERGY for exponential tail on resolution broadening | See Section III.C.1.c; the energy shift ΔEs is assumed zero here. | 235 |
|  |  | PLOT RESOLUTION FUNCtion | See Section X.L. This command is used to make plots of the original (Gaussian and/or exponential) resolution function. | 236 |
|  |  | RPI DEBUG | Used to obtain extra printouts when debugging the RPI resolution function. | 239 |
|  |  | CENTER THE CONSTANT energy resolution function | See Section III.C.4. | 240 |
| Broadening – resolution (cont.) |  | (No command) | When using the RPI/GELINA/ nTOF resolution function, the user must specify whether or not the centroid of the resolution function is to be shifted. |  |
|  |  | SHIFT RPI RESOLUTION function to center, or SHIFT GEEL RESOLUTIOn function to center, or SHIFT GELINA RESOLUTion or SHIFT NTOF RESOLUTIOn | The RPI/GELINA/nTOF resolution function will be centered so as not to shift the peak energies of resonances. See Section III.C.3. | 237 241 243 245 |
|  |  | DO NOT SHIFT RPI RESolution function to center, or DO NOT SHIFT GEEL REsolution function to center, or DO NOT SHIFT GELINA resolution function to center, or DO NOT SHIFT NTOF REsolution function to center | The RPI/GELINA/nTOF resolution function is to be used exactly as input with no centering. | 238 242 244 246 |