VI.  INPUT TO SAMMY

Input to SAMMY consists of three or more files, which may be user generated or (sometimes) produced by earlier SAMMY runs.  These files are summarized in Table VI.1 and described in detail in the sections listed in the table.  Test case numbers (see Section XII.B) are given for those files that are infrequently used.

Table VI.1.  SAMMY input files

| Description | Contents | Section(s) / table(s) containing details | Test case using this file |
| --- | --- | --- | --- |
| Interactive input | Interactive input to SAMMY (file names, energy ranges) | Section VI.E |  |
| INPut file | Details about the interaction being studied, including SAMMY control information | Section VI.A Tables VI A.1 and VI A.2 |  |
| PARameter file | Initial values for resonance and other parameters, flags defining which are to be varied, uncertainty information | Section VI.B Table VIB.2 |  |
| DATa file | Experimental data, including energy, cross section (or transmission, for example), and uncertainty | Section VI.C.1 Table VI C1.1 |  |
| COVariance file | Covariance matrix for the parameters as output by an earlier SAMMY run | Section VII.B |  |
| DCV (Data CoVariance) | Covariance information for the data | Section VI.C.2 | tr007, tr118 |
| IDC (Implicit Data Covariance) for normalization and backgrounds only | Information on data-reduction parameters to be used to generate implicit data covariance matrix | Section VI.C.3.c | tr070 |
| IDC (User supplied) | Information on data-reduction parameters to be used to generate implicit data covariance matrix | Section VI.C.3.b | tr118 tr140 (*.dcv) |
| NTG file | Integral data file | Section VI.D | tr069, tr077 |
| AVG file or BON file | Information needed by SAMMY in order to produce energy-averaged or Bondarenko-averaged cross sections | Section VI.F.1 | tr014, tr083, tr084, tr085 |
| NDF file | Information needed by SAMMY in order to generate an output file in ENDF-6 File 2 format | Section VI.F.2 | tr023 |
| ENDF resonance parameter file | File 2 from ENDF, to be used instead of the usual SAMMY PARameter file (and for part of the SAMMY INPut file) | Section VI.F.3 | tr049, tr050, tr086, tr091, tr102, tr129 |
| ENDF File 3 | Smooth cross sections to be added to the cross section calculated from resonance parameters, for use in calculating the stellar (Maxwellian) averages | Section V.D | tr051 |
| MXW file | Temperatures at which the stellar (Maxwellian) averages are to be evaluated | Sections V.D and VI.F.4 | tr042, tr049, tr051 |
| SSM file | Edge-effects corrections to single-scattering correction for capture cross sections (file is generated by a previous SAMMY run) | Section III.D | tr039 and many others |
| URR PAR file | Input information for unresolved resonance region | Section VIII | tr073, tr088 |
| YWY file | W and Y matrices from a single data set, when analyzing several data sets simultaneously, e.g., for creating a retroactive parameter covariance matrix (files are generated by previous SAMMY runs) | Sections IV.E.1 and IV.E.2 | tr082, tr089, tr131, tr149 |